A 12

Sreenidhi Institute of Science & Technology

(An Autonomous Institution)

Code No:  121EC05

B. TECH. I – Year II – Semester Examinations, July, 2014 (Regular)

ELECTRONIC DEVICES AND CIRCUITS (ECE)

Time:	3 Hours								     		          Max. Marks: 70

Note: No additional answer sheets will be provided.

Part-A

Max.Marks:20

Answer all QUESTIONS.

- Compare electro static and electromagnetic deflection mechanism.

- Define static and dynamic resistances.

- List the biasing techniques employed in transistor.

- What are the advantages of FET over BJT?

- Write the differences between small signal and large signal amplifier.

- Define voltage regulator and voltage multiplier.

- Draw the Zener diode characteristics.

- What is the condition of thermal stability?

- Draw the frequency response of RC- Coupled amplifier.

- Draw a diagram giving basic structure of UJT.

Part – B

Max. Marks: 50

ANSWER ANY FIVE QUESTIONS. EACH QUESTION CARRIES 10 MARKS.

- a) Describe the method of electro static focusing in a cathode ray tube.            		 [6M]

b) An electron is emitted from a thermionic cathode with zero initial velocity and at a potential of 300V. Calculate the final velocity.                                                           			[4M]

2)	a) Explain breakdown mechanism in diodes.                                                                	[4M]

b)The reverse  saturation current at 300k of a germanium diode is 5nA. Determine the voltage to 	be applied to obtain a forward current of 50mA.                                         			 [6M]

3)  	a) Derive expression for the stability factor in self bias method.                        		 [6M]

b) Draw the h- parameter model of transistor in CE configuration and describe its parameters.                                                                                                                            													[4M]

4)	a) Write the differences between BJT and FET.                                                       	[4M]

b) Draw and discuss volt – ampere characteristics of SCR.                                  		[6M]

5)	a) Compare CE,CB,CC amplifiers.                                                                                 	[4M]

b) Explain about RC- coupled amplifier.                                                                     	[6M]

6)	a) Draw the Schematic diagram of a voltage doubler  and explain its working.  		[4M]

b) Draw the circuit for series type voltage regulator and explain its working. 		[6M]

7)	a) A CC amplifier is driven by a voltage source of 0.1V, internal resistance of 1000 Ω the load 	impedance of 1000 Ω  and h- parameters are hie= 1100Ω,hre =2.5x10-4 ,hfe = 50,& hoe =25µA/V 	then compute Ai , Zi  Av , Yo.                                         						[6M]

b) Derive expression for magneto static deflection sensitivity in cathode ray tube.		[4M]

8)	a) Explain the operation of n- channel  depletion mode MOSFET.                           	[6M]

b) Write short notes on short circuit protection.                                                           	[4M]

-- 00 -- 00 --