[image: image1.emf]V

0

-

+

A

3

2

6

-V

EE

V

CC

4

7

V

1

=2V

50KΩ 

V

2

=3V

V

3

=4V

V

4

=5V

40KΩ 

25KΩ 

10KΩ 

20KΩ 

3

0

K

Ω

 


Sreenidhi Institute of Science and Technology

(An Autonomous Institution)
Code No:  3C513
B. TECH. III-Year I-Semester Examinations, December 2015 (Supplementary)

LINEAR AND DIGITAL IC APPLICATIONS 

(Common to EEE, ECE & ECM)

Time:
3 Hours







      Max. Marks: 70


 Note: No additional answer sheets will be provided.

Part - A 

Max.Marks:20

Answer all QUESTIONS. EACH QUESTION CARRIES 2 MARKS.

1.  The common mode input to a certain differential amplifier having differential gain of   

     125 is 4sin 200πt Volts. Determine the common mode output, if CMRR is 60dB?

2.  Draw the frequency response curve of a differentiator. How it is modified when a  

     small resistor is connected in series with capacitor?
3. Calculate the value of resistance required in a second order low pass Butterworth  filter having cut-off frequency of 15KHz, with a capacitor of 0.02µf ?

4.  Why is the R-2R ladder network DAC is better than weighted resistor DAC?

5.  Draw the circuit diagram of CMOS inverter and explain its operation using truth table. 

6.  Draw the logic diagram and truth table of 2 to 4 decoder. 

7. Define slew rate? What causes the slew rate? Write the expression for maximum frequency of slew rate?

8. Illustrate the advantages of active filters over passive filters.

9. List out the important features of Emitter Coupled Logic (ECL).

10. Draw the truth table and logic diagram of Master Slave JK Flip Flop.

Part – B

Max.Marks:50

ANSWER ANY FIVE QUESTIONS. EACH QUESTION CARRIES 10 MARKS.

1. 

a) List the non-ideal DC characteristics of an OP-AMP. Explain any two 
parameters in detail?

     
b) An op-amp with a slew rate = 0.5V/μs is used as an inverting amplifier to 
obtain a gain of 100. The voltage gain Vs frequency characteristic of the 
amplifier is flat up to 10 KHz. Determine 

i) The maximum peak-to-peak input signal that can be applied without any distortion to the output .

ii) The maximum frequency of the input signal to obtain a sine wave output of 2V peak.






2. 
a) Draw the circuit diagram of instrumentation amplifier used to measure the 
temperature and  explain its working in detail by deriving output expression?








b) Find the output voltage for the circuit shown below:


[image: image4.jpg]Sreenidhi Institute of Science and Tm:hnnlngy

AUTONOMOUS





3. 
a) Derive the transfer function, gain, and phase angle for first and second 
order 
low pass active filters.











b) Design a PLL circuit using 565IC to get free running frequency of 4.5KHz with 
lock range of 2KHz and capture range of 100Hz. Assume supply voltages of 

[image: image2.wmf]±

10V. Also draw the circuit diagram.









                        

4. 
a) Draw the block diagram of 4-bit successive approximation A/D converter and 
explain its operation with the help of output waveform.  


b) Design a RC phase shift oscillator for 300Hz frequency using IC741 and 

[image: image3.wmf]±

15V 
power supply?








5. 
a) Briefly describe the characteristics of TTL and CMOS logic families.   




b) Describe tri-state logic (TSL) using TTL logic family. Draw and explain the 
circuit of a TSL NAND gate.





6. 
a) Differentiate between a multiplexer and a demultiplexer. Using two 74151s, 
design a 16-line multiplexer controlled by four data select control inputs.  




b) Design a 6-bit adder using two 74283 ICs. 







7. 
a) Draw the block diagram of IC 566 VCO and explain its operation.




b) An astable multivibrator has RA=2.2KΩ, RB=6.8KΩ and C=0.01µf. Calculate


    i) tHIGH
ii) tLOW

iii) free running frequency
iv) Duty cycle














8. 
a) Explain the working of a weighted resistor D/A converter and state the 
limitations of it.







b) Explain in detail the working of a positive edge triggered D-type flip-flop.
 




-- 00 -- 00 –




A12








PAGE  
Page 1 of 2

_1511082219.unknown

_1511082220.unknown

_1511082218.vsd
V0


-


+


A


3



