[image: image1.jpg]Sreenidhi Institute of Science and Tm:hnnlngy

AUTONOMOUS





Sreenidhi Institute of Science and Technology

(An Autonomous Institution)
Code No:  3B513
B. TECH. III-Year I-Semester Examinations, December 2015 (Regular)

APPLIED THERMODYNAMICS-II (ME)
Time:
3 Hours







      

     Max. Marks: 70


 Note: No additional answer sheets will be provided.

Part - A 
Max.Marks:20

Answer all QUESTIONS. EACH QUESTION CARRIES 2 MARKS.
1)
Why Rankine cycle is considered as a Ideal cycle?
2) 
What is Boiler Draught ? Give classification.

3) 
What do you understand by critical pressure ratio in nozzles?
4)
How are turbines classified?

5) 
What is compound of turbines.. why it is done?

6) 
What is the requirement of a condenser in a steam power plant.?

7)
What is inter cooling in Gas turbines?

8) 
What is the difference between Jet Propulsion and Rocket Engine?

9)
What is the Stage efficiency of a turbines?

10)
What do you understand by Reheating in Rankine cycle?
Part – B
Max.Marks:50
ANSWER ANY FIVE QUESTIONS. EACH QUESTION CARRIES 10 MARKS.
1)
 A simple Rankine cycle works between pressures of 28bar and 0.06bar, the initial condition of 
the 
steam being dry saturated. Calculate the cycle efficiency , work ratio and specific steam 
consumption.

2) 
Explain about Benson boiler with a neat sketch and give its advantages.

3)
 a) What is super-saturated flow in nozzles ..?





(3M+7M)
      
 b) Steam enters a steam nozzle at a pressure of 15bar, 2500C and is discharged to a pressure of                   
 2 bar. Neglect initial velocity of steam. If 10% of heat drop is lost in friction, if the exit diameter is 
 

            300mm determine 


i) 
Mass flow rate 


ii) 
Throat area 



iii) 
Exit velocity of steam
4) 
A single stage steam turbine is supplied with a steam velocity of 1000m/sec. The blade speed is 
400m/sec. The  nozzles are inclined at an angle of 200 to the plane of the wheel and the outlet blade 
angle 
is 300. Neglecting frictional losses, determine the power developed , blade efficiency, 
and 
stage efficiency if nozzle efficiency is 93%.

5) 
In a gas turbine plant, the compressor takes in air at a temperature of 150C and compresses it to four 
times the initial pressure with an isentropic efficiency of 85%. The air is then passed through a heat 
exchanger, heated by the turbine exhaust before reaching the combustion chamber. The turbine inlet 
temperature is 6000C and its efficiency is 80%.  Neglecting all losses except those mentioned, and 
treating the working fluid throughout the cycle to have the properties of air, calculate thermal efficiency 
and work ratio of the cycle if heat exchanger gives 85% of available heat to air.


Take R = 0.287 Kj/Kg. K,  ( = 1.4, and constant specific heats throughout.

6) 
 Air enters at the rate of 15 kg/s into a compressor of jet aircraft travelling at 241 m/s. The air fuel ratio 
is 60:1 and the compression pressure ratio is 6:1. The calorific value of the fuel is 
41820kJ/kg. When 
enthalpy changes for the nozzle is 200 Kj/Kg and velocity coefficient is 
0.97. Neglecting all losses 
calculate the thrust, thrust power, specific thrust consumption and propulsive efficiency.
7)
Explain the various types of condensers with neat sketches
8) 
Write short notes on:









(7M+3M)


i) 
Regeneration in rankine cycle.




ii) 
Wilsons line in steam nozzels.
-- 00 -- 00 –



A12








PAGE  
Page 2 of 2

