[image: image1.jpg]Sreenidhi Institute of Science and Tm:hnnlngy

AUTONOMOUS






Sreenidhi Institute of Science and Technology

(An Autonomous Institution)
Code No: 5B626      





                  Date: 29-Nov-2018 (AN)
B.Tech III-Year II-Semester External Examination, November - 2018 (Supplementary)

Automobile Engineering (ME)
Time:
3 Hours







                    Max.Marks:75

 
Note:    a)  No additional answer sheets will be provided.

          

b)  All sub-parts of a question must be answered at one place only, otherwise it will not be valued.

          

c)  Missing data can be assumed suitably.








Part - A 



Max.Marks:25
Answer all QUESTIONS.
	1.
	Define chassis.
	[3M]

	2.
	What are the difficulties of simple carburetor?
	[3M]

	3.
	List the components of water cooling system.
	[3M]

	4.
	What are various alternative fuels available for running an Automobile?
	[3M]

	5.
	What is the main function of clutch?
	[3M]

	6.
	Define: Toe in and Toe out.
	[2M]

	7.
	List various Lubricating oil pumps used in Automobile.
	[2M]

	8.
	Describe the function of Contact breaker in Ignition system of an Automobile.
	[2M]

	9.
	Enumerate the requirements of Braking fluid.
	[2M]

	10.
	What are the different types of steering gears ?
	[2M]








Part – B



   
 Max.Marks:50
ANSWER ANY FIVE QUESTIONS. EACH QUESTION CARRIES 10 MARKS.
	11.
	a)
	Discuss the advantages and disadvantages of front wheel drive.
	[6M]

	
	b)
	Describe the operation of splash lubrication system with neat sketch.
	[4M]

	
	
	
	

	12.
	
	Describe the operation of distributor type Fuel pump with neat sketch.
	[10M]

	
	
	
	

	13.
	a)
	Describe the working of spark plug.
	[6M]

	
	b)
	Explain the use of condenser in Ignition system.
	[4M]

	
	
	
	

	14.
	a)
	Give details of Bendix drive Mechanism.
	[5M]

	
	b)
	Explain Horn with neat sketch.
	[5M]

	
	
	
	

	15.
	a)
	Describe constant Mesh Gear box with neat sketch.
	[6M]

	
	b)
	Explain about over drive torque converter.
	[4M]

	
	
	
	

	16.
	a)
	Explain rigid axle steering Linkage system.
	[6M]

	
	b)
	write any FOUR advantages of Independent suspension systems over rigid axle

suspension system
	[4M]

	
	
	
	

	17.
	
	Explain Zenith Carburetor with neat sketch.
	[10M]

	
	
	
	

	18.
	
	Explain the operation of single plate clutch and write its advantages and disadvantages
	[10M]


-- 00 -- 00 –
H.T No





Regulations:


A15











PAGE  
Page 1 of 1

