[image: image6.jpg]Sreenidhi Institute of Science and Tm:hnnlngy

AUTONOMOUS






Sreenidhi Institute of Science and Technology

(An Autonomous Institution)
Code No: 5B730    






                  Date: 22-Nov-2018 (FN)
B.Tech IV-Year I-Semester End External Examination, November - 2018 (Regular)

Finite Element Method (ME)
Time:
3 Hours







                    Max.Marks:75

 
Note:    a)  No additional answer sheets will be provided.

          

b)  All sub-parts of a question must be answered at one place only, otherwise it will not be valued.

          

c)  Missing data can be assumed suitably.








Part - A 



Max.Marks:25
Answer all QUESTIONS.
	1.
	Discuss the history of development of Finite element method.
	[3M]

	2.
	Write the transformation matrix in truss members to convert local and global displacement vectors.
	[3M]

	3.
	What is meant by iso-parametric representation?
	[3M]

	4.
	Sketch the shape functions for a two dimensional four node quadrilateral element.
	[3M]

	5.
	List the boundary conditions of steady state one dimensional conduction heat transfer problems
	[3M]

	6.
	List some FEA software packages.






	[2M]

	7.
	Discuss the load vector for uniformly distributed load on beam element.
	[2M]

	8.
	Write the strain displacement relations in axi-symmetric solid element.  
	[2M]

	9.
	Write the load vector due to convection at the boundary in one dimensional heat transfer
	[2M]

	10.
	Differentiate dynamic and static analyses.
	[2M]







Part – B



   
 Max.Marks:50
ANSWER ANY FIVE QUESTIONS. EACH QUESTION CARRIES 10 MARKS.
	11.
	
	Axial load P = 300 KN is applied at 20° C to the rod as shown in Fig.1. The temperature is then raised to 60° C. The coefficient of thermal expansion for Aluminum is 23x10-6 per °C and Steel is 11.7x10-6 per °C. AAl = 900 mm2 , ASteel = 1200 mm2 , EAl = 70 x 109 N/m2 ,  ESteel = 200 x 109 N/m2 . Using FEM, Determine 

i) the nodal displacement and 


ii) element stresses and the reaction forces at the supports.
[image: image1.png]200 mm_

Aluminium

Steel










Fig.1
	[10M]

	
	
	
	

	12.
	
	For the two-bar truss shown in Fig.2, A force of P =1000 kN is applied at node 1 in the positive y direction while node 1 settles an amount δ = 50 mm in the negative x direction. Let E = 210 GPa and A = 6.00 x 10-4 m2 for each element. The lengths of the elements are shown in the figure. 

determine i) the displacement in the y direction of node 1 and 


     ii) the axial force in each element.
[image: image2.png]






     Fig.2
	[10M]

	
	
	
	

	13.
	
	The co-ordinates of an axi-symmetric element in mm are shown in Fig.3. Evaluate

i. Jacobian matrix.


ii. Strain-displacement matrix.

z


 3(30,40)


1(20,20)
2(40,20)

                                                        Fig.3          r

	[10M]

	
	
	
	

	14.
	
	For the iso-parametric quadrilateral element shown in Fig.4, with the Cartesian co-ordinates.in cm. Evaluate

i).Shape functions.

ii). Jacobian matrix.

[image: image3.png]wo

s

a0

s



[image: image4.png]



Fig-4
	[10M]

	
	
	
	

	15.
	
	Derive the finite element equations for steady state one dimensional fin problem by using Galerkin’s method.
	[10M]

	
	
	
	

	16.
	
	Determine the eigen values of a stepped bar shown in Fig.5,
                                         A1=100 mm2
         A2=40 mm2




400m

Fig.5            800mm
	[10M]

	
	
	
	

	17.
	a)
	Explain the penalty approach of imposing boundary conditions
	[5M]

	
	b)
	Develop the shape functions for a beam element and show them.
	[5M]

	
	
	
	

	18.
	a)
	Evaluate the integral by two point Gaussian quadrature,
[image: image5.png]= [ L@+ 32y + 4y? Ydxdy




	[5M]

	
	b)
	Explain Convergence criterion.
	[5M]


-- 00 -- 00 –
H.T No





Regulations:


A15











PAGE  
Page 2 of 2

