[image: image8.jpg]Sreenidhi Institute of Science and Tm:hnnlngy

AUTONOMOUS






Sreenidhi Institute of Science and Technology

(An Autonomous Institution)
Code No: 7BC03 




                  

     Date: 24-June-2019 (AN)
B.Tech I-Year I-Semester External Examination, June-2019 (Supplementary)

ENGINEERING MECHANICS (ME)
Time:
3 Hours







                    Max.Marks:70

 
Note:    a)  No additional answer sheets will be provided.

          

b)  All sub-parts of a question must be answered at one place only, otherwise it will not be valued.

          

c)  Missing data can be assumed suitably.








Part - A 



Max.Marks:20
Answer all QUESTIONS.
	1.
	Determine the resultant of an 800N force acting towards eastern direction and a 500N force acting towards north eastern direction.
	[2M]

	2.
	Define the principle of virtual work.
	 

	3.
	Define principal axes and principal moment of inertia.
	[2M]

	4.
	Differentiate between “Mass moment of inertia” and  “Area moment of inertia”
	[2M]

	5.
	A point P moves along a straight line according to the equation,  x= 4t3 +2t+5, where x is in meters and t is in seconds. Calculate the velocity and acceleration at t=3 seconds.
	[2M]

	6.
	Define Co-efficient of restitution.
	[2M]

	7.
	How do you represent the inclined roller support in structures in free body diagram.
	[2M]

	8.
	What is meant by pivot friction?
	[2M]

	9.
	Define conservation of linear momentum.
	[2M]

	10.
	A semicircular area having radius of 100 mm is located in the XY plane such that its diameter coincides with the Y-axis. Determine the X-coordinate of the centroid.
	[2M]










       Part – B



    Max.Marks:50
ANSWER ANY FIVE QUESTIONS. EACH QUESTION CARRIES 10 MARKS.
	11.
	
	A force P is applied at θ0 to the string AOB as shown in Fig.1. If the tension in each part of string is 50 N, Find the direction and magnitude of force P for equilibrium conditions.

[image: image1.png]



Fig.1
	[10M]

	
	
	
	

	12.
	
	Determine the force in each member of the truss in Fig.2 and indicate if the members are in tension or compression.

[image: image2.png]



    






Fig.2
	[10M]

	
	
	
	

	13.
	
	Determine the centroid ([image: image4.png]


 ) of the composite area shown in Fig.3

[image: image5.png]



Fig.3
	[10M]

	
	
	
	

	14.
	
	Determine the moment of inertia of the composite area shown in Fig.4 about the centroidal y-axis.


[image: image6.png]



    





Fig.4
	[10M]

	
	
	
	

	15.
	a)
	Discuss the rigid body fixed axis rotation.
	[5M]

	
	b)
	A boy drops a stone from the top of well vertically downwards into it. The splash is heard by him after 6 seconds. Find the well depth by taking sound velocity as 400 m/s.
	[5M]

	
	
	
	

	16.
	
	In Fig.5, If the coefficient of kinetic friction between the 100-kg crate and the plane µk=0.25, determine the speed of the crate at the instant the compression of the spring is x=1.5 m. Initially the spring is un-stretched and the crate is at rest.

[image: image7.png]



Fig.5
	[10M]

	
	
	
	

	17.
	
	Explain about:
	

	
	a)
	The equilibrium of co-planar force system.
	[5M]

	
	b)
	Wedge friction.
	[5M]

	
	
	
	

	18.
	
	Discuss about:
	

	
	a)
	Transfer formula for moment of inertia.
	[5M]

	
	b)
	Elastic impact.
	[5M]


-- 00 -- 00 –
H.T No





Regulations:


A18











PAGE  
Page 2 of 2

_1622887099

