[image: image6.png]



[image: image7.jpg]Sreenidhi Institute of Science and Tm:hnnlngy

AUTONOMOUS





Sreenidhi Institute of Science and Technology

(An Autonomous Institution)
Code No: 4AC41 





                  
        Date: 11-June-2019 (FN)
B.Tech I-Year II-Semester External Examination, June-2019 (Supplementary)

BASIC Electrical Engineering (BT)
Time:
3 Hours







                    Max.Marks:70

 
Note:    a)  No additional answer sheets will be provided.

          

b)  All sub-parts of a question must be answered at one place only, otherwise it will not be valued.

          

c)  Missing data can be assumed suitably.








Part - A 



Max.Marks:20
Answer all QUESTIONS.
	1.
	State Ohm’s Law and write its limitations.
	[2M]

	2.
	Explain the terms RMS  value, Form Factor and Peak factor.
	[2M]

	3.
	Derive the torque equation of a D.C Motor.
	[2M]

	4.
	Define regulation of a transformer?
	[2M]

	5.
	Obtain the relation between phase voltages and line voltages in a delta connected system.
	[2M]

	6.
	List the advantages of PMMC instruments.
	[2M]

	7.
	State Faraday’s Laws of Electromagnetic Induction.
	[2M]

	8.
	A 220V, D.C. series generator has constant loss of 300W and an armature resistance of 0.15Ω. Determine the load current at maximum efficiency.
	[2M]

	9.
	A three phase induction motor is wound for 4 poles and is supplied from 50Hz supply. Calculate (i) Synchronous speed, (ii) rotor speed when running at a slip of 4%.
	[2M]

	10.
	State Thevenin’s Theorem.
	[2M]









            Part – B


   
   Max.Marks:50
ANSWER ANY FIVE QUESTIONS. EACH QUESTION CARRIES 10 MARKS.
	11.
	a)
	State and explain Kirchhoff's laws.
	[5M]

	
	b)
	Determine the current through 1Ω resistor in Fig. 1 using mesh analysis.


[image: image1.png]Rov





Fig. 1
	[5M]

	
	
	
	

	12.
	a)
	Define the following terms:

 (i) Instantaneous value           (ii) Cycle

(iii) Time period
(iv) Frequency
	[4M]

	
	b)
	A two element series circuit is connected across an A.C source            [image: image3.png]v(t) = 200v2sin(314t +20°) V.



 
The current in the circuit is then found to be  [image: image5.png]i(£) = 10V2 cos (314t — 25°



)A. 
Determine the parameters of the circuit.
	[6M]

	
	
	
	

	13.
	a)
	Derive the EMF equation of a D.C generator.
	[4M]

	
	b)
	A 4 pole, lap wound D.C shunt generator has a useful flux per pole of 0.07Wb. The armature winding consists of 220 turns with an effective resistance of 0.1Ω. Calculate the terminal voltage when running at a speed of 900rpm and drawing an armature current of 50A.
	[6M]

	
	
	
	

	14.
	
	A 10kVA, 450/120V, 50Hz single phase transformer gave the following test results:

Open circuit test: Vo = 120V, Io = 4.2A, Wo = 80W
Short circuit test: Vsc = 9.65V, Isc = 22.2A, Wsc = 120W                                                  

i) Find the equivalent circuit referred to Low voltage side.

ii) Determine the voltage regulation at 80% full load, 0.85 power factor lagging.
	[10M]

	
	
	
	

	15.
	
	Derive the expression for total power input for a star connected balanced three phase load interms of line voltage, line current and power factor with the help of respective phasor diagrams.
	[10M]

	
	
	
	

	16.
	a)
	Discuss about the construction and principle of operation of Moving Iron(MI) instruments with neat sketch.
	[6M]

	
	b)
	Write the advantages and disadvantages of MI instruments.
	[4M]

	
	
	
	

	17.
	a)
	Determine the current “I” in the network shown in Fig. 2 using Superposition Theorem.                             

Fig. 2
	[6M]

	
	b)
	Explain about the internal and external characteristics of a D.C shunt generator.
	[4M]

	
	
	
	

	18.
	a)
	Explain the operation of a transformer on no – load and on – load with necessary phasor diagrams.
	[6M]

	
	b)
	Explain how the range of an ammeter is extended using necessary equations.
	[4M]


-- 00 -- 00 –
H.T No





Regulations:


A14











PAGE  
Page 2 of 2

_1621750977

