[image: image4.jpg]Sreenidhi Institute of Science and Tm:hnnlngy

AUTONOMOUS






Sreenidhi Institute of Science and Technology

(An Autonomous Institution)
Code No: 4A243/4AC43




                  
        Date: 18-June-2019 (FN)
B.Tech I-Year II-Semester External Examination, June-2019 (Supplementary)

NETWORK ANALYSIS (ECE and ECM)
Time:
3 Hours







                    Max.Marks:70

 
Note:    a)  No additional answer sheets will be provided.

          

b)  All sub-parts of a question must be answered at one place only, otherwise it will not be valued.

          

c)  Missing data can be assumed suitably.








Part - A 



Max.Marks:20
Answer all QUESTIONS.
	1.
	Differentiate between ideal voltage source and practical voltage source.
	[3M]

	2.
	Differentiate between planar and non-planar graph with suitable example.
	[3M]

	3.
	Define form factor and peak factor of an alternating quantity.
	[3M]

	4.
	State Milliman’s Theorem.
	[3M]

	5.
	Draw the Y-parameter equivalent circuit.
	[3M]

	6.
	Draw the curves showing the transient current and transient voltages across resistor and inductor in a R-L series circuit when connected to a constant voltage source.
	[2M]

	7.
	What is a dual network?
	[2M]

	8.
	Explain the concept of mutual inductance.
	[2M]

	9.
	Mention the symmetrical conditions for h-parameters of a two port network.
	[2M]

	10.
	Explain the concept of super node in network analysis.
	[2M]









            Part – B


   
   Max.Marks:50
ANSWER ANY FIVE QUESTIONS. EACH QUESTION CARRIES 10 MARKS.
	11.
	a)
	Explain the mesh analysis used in the circuit analysis.
	[5M]

	
	b)
	Explain the classification of electrical networks.
	[5M]

	
	
	
	

	12.
	a)
	Explain the procedure for obtaining fundamental cut-set matrix of a given network.
	[5M]

	
	b)
	What is an incidence matrix? What are the properties of complete incidence matrix?
	[5M]

	
	
	
	

	13.
	a)
	Explain about the sinusoidal response of R-C series circuit and derive necessary equations?
	[5M]

	
	b)
	Derive the expression for Bandwidth of a series resonant circuit and explain with the help of sketches how bandwidth varies with quality factor.
	[5M]

	
	
	
	

	14.
	
	Obtain the voltage between the terminals A and B in the circuit shown in figure 1, using superposition theorem

[image: image1.emf]
Figure 1


	[10M]

	
	
	
	

	15.
	a)
	Obtain the expressions for h-parameters of a two port network.
	[5M]

	
	b)
	Derive the condition of symmetry for short circuit parameters.
	[5M]

	
	
	
	

	16.
	a)
	Explain the transient response of a R-C series circuit with sinusoidal voltage applied to it.
	[5M]

	
	b)
	Explain the significance of initial conditions in the dc response of RC and RL networks.
	[5M]

	
	
	
	

	17.
	
	Using mesh analysis, calculate the current I1 shown in figure 2.

[image: image2.emf]
Figure 2
	[10M]

	
	
	
	

	18.
	
	Using Thevenin’s theorem, obtain the current Ix in the circuit shown in figure 3.

[image: image3.emf]
Figure 3
	[10M]


-- 00 -- 00 –
H.T No





Regulations:


A14











PAGE  
Page 2 of 2

