H.T No

Regulations:

A15

Sreenidhi Institute of Science and Technology

(An Autonomous Institution)

Code No: 5BC11     							        Date: 10-June-2019 (FN)

B.Tech II Year II-Semester External Examination, May/June-2019 (Supplementary)

OPERATIONS RESEARCH (ME)

Time:	3 Hours								                    Max.Marks:75

Note:    a)  No additional answer sheets will be provided.

b)  All sub-parts of a question must be answered at one place only, otherwise it will not be valued.

c)  Missing data can be assumed suitably.

Part - A 				Max.Marks:25

Answer all QUESTIONS.

| 1. | What are slack and surplus variables? | [3M] |
| --- | --- | --- |
| 2. | Explain about Hungarian Method in Assignment problem. | [3M] |
| 3. | Write steps to solve n jobs through two machines problem | [3M] |
| 4. | What is saddle point in Game theory? | [3M] |
| 5. | Define EOQ in Inventory Model. | [3M] |
| 6. | What are the types of simulation? | [2M] |
| 7. | What is Degeneracy in Simplex Method? | [2M] |
| 8. | What are the situations for replacement? | [2M] |
| 9. | What is Dominance principle in Theory of Games? | [2M] |
| 10. | What is Queue behavior? | [2M] |

Part – B			   	 Max.Marks:50

ANSWER ANY FIVE QUESTIONS. EACH QUESTION CARRIES 10 MARKS.

| 11. | a) | Maximize Z=2x-y+z s.t 3x-2y+2z≤15, -x+y+z≤3 x-y+z≤4 x,y,z≥0 | [5M] |
| --- | --- | --- | --- |
|  | b) | Write about scope and Phases of OR | [5M] |
| 12. | a) | Solve the following Transportation problem using MODI method. D1; D2; D3; D4; Supply. S1; 19; 30; 50; 10; 7. S2; 70; 30; 40; 60; 9. S3; 40; 8; 70; 20; 18. Demand; 5; 8; 7; 14. | [5M] |
|  | b) | Solve the following travelling salesman problem. TO. From; A; B; C; D; E. A; -; 4; 10; 14; 2. B; 12; -; 6; 10; 4. C; 16; 14; -; 8; 14. D; 24; 8; 12; -; 10. E; 2; 6; 4; 16; -. | [5M] |
| 13. |  | Find the sequence of jobs that minimizes the total elapsed time to complete the following jobs on two machines. Jobs; 1 2 3 4 5 6. Machine A Machine B; 3 12 5 2 9 11 8 10 9 6 3 1. | [10M] |
| 14. |  | Solve the following game 172027516 | [10M] |
| 15. |  | Patients arrive at a clinic in a poisson manner at an average rate of 6 per hour. The doctor on average can attend to 8 patients per hour. Assuming that the service time distribution is exponential, find (i) Average number of patients waiting in the queue. (ii) Average time spent by a patient in the clinic. | [10M] |
| 16. | a) | Write about Bellman’s principle of Optimality. | [5M] |
|  | b) | Write about MONTE-CARLO Technique. | [5M] |
| 17. |  | A machine costs Rs. 12,200. The scrap value is RS. 200. The maintenance costs of the machine are given below Year; 1 2 3 4 5 6 7 8. Maintenance Cost; 200 500 800 1200 1800 2500 3200 4000. When should the machine be replaced. | [10M] |
| 18. |  | A manufacturer has to supply 12000 units of a product per year to his customer. Shortages are not permitted and there is no lead time. The inventory holding cost is Rs. 0.20 per unit per month and the set-up cost per run is Rs. 350. Determine (i) the economic lot size (ii) the period of one run (iii) the minimum annual inventory cost. | [10M] |

-- 00 -- 00 –