[image: image6.jpg]Sreenidhi Institute of Science and Tm:hnnlngy

AUTONOMOUS






Sreenidhi Institute of Science and Technology

(An Autonomous Institution)
Code No: 5K502      





                  Date: 30-May-2019 (AN)
B.Tech III-Year I-Semester External Examination, May/June-2019 (Supplementary)

Structural Analysis - II (CIVIL)
Time:
3 Hours







                    Max.Marks:75

 
Note:    a)  No additional answer sheets will be provided.

          

b)  All sub-parts of a question must be answered at one place only, otherwise it will not be valued.

          

c)  Missing data can be assumed suitably.








Part - A 



Max.Marks:25
Answer all QUESTIONS.
	1.
	A UD live load of 60KN/m run of length 5m moves on a girder of span 16m, find  maximum positive and negative shear force at a section 6m from the left end.
	[3M]

	2.
	Derive an expression for effect of shortening of rib in a two hinged parabolic arch.
	[3M]

	3.
	Write a short note on temperature stresses in two-hinged girder.



	[3M]

	4.
	Develop flexibility matrix for the beam AB shown in fig. EI is constant throughout.


[image: image1.png]



	[3M]

	5.
	Define stiffness matrix.


	[3M]

	6.
	State assumptions in plastic theory.
	[2M]

	7.
	Name the type of rolling loads for which the absolute maximum bending moment occurs at the mid span of a beam.
	[2M]

	8.
	Define plastic hinge.
	[2M]

	9.
	A simply supported beam of rectangular section carries a uniformly distributed load over the whole span. Determine at collapse condition, what part of the beam is fully elastic.
	[2M]

	10.
	Determine the horizontal thrust developed in the semi-circular arch of radius 18m. loaded as shown in fig.

[image: image2.png]60KN
C 40KN

18m 18m




	[2M]









Part – B


   
 Max.Marks:50
ANSWER ANY FIVE QUESTIONS. EACH QUESTION CARRIES 10 MARKS.
	11.
	
	Five wheel loads 10KN, 20KN, 15KN, 30KN and 18KN spaced at 0.8m interval roll on a girder of span 25m from left to right with the 10KN load leading. Find 

i. Absolute maximum bending moment for girder.

ii. Greatest reaction at A

iii. Bending moment at a section 10m from left support.
	[10M]

	
	
	
	

	12.
	
	A two hinged semi-circular arch of span 20m is subjected to uniformly distributed load of 50KN/m over the whole span and a concentrated load of 80KN at 5m from right support. Determine the horizontal thrust.
	[10M]

	
	
	
	

	13.
	
	A cable of span 20m and dip 4m carries a uniformly distributed load of 20KN/m over the whole span. Find 










i. Maximum tension in the cable.

ii. Minimum tension in the cable

iii. The length of the cable.
	[10M]

	
	
	
	

	14.
	
	Analyze the frame shown in fig. by force method. Draw bending moment diagram.



[image: image3.png]2.5m

(31

32KN

1.5m ¢

(21

3m

3m

18KN




	[10M]

	
	
	
	

	15.
	
	Analyze the frame shown in fig. by displacement method.





[image: image4.png]6KN 6KN





	[10M]

	
	
	
	

	16.
	
	Find the plastic moment of resistance required for a propped cantilever of span 5m to carry a safe concentrated load of 50KN at a distance of 3m from the fixed end. Take load factor=1.7.

	[10M]

	
	
	
	

	17.
	
	Determine the influence line ordinates at 1m interval for moment at mid- span of BC of the continuous beam shown in fig. 


[image: image5.png]



	[10M]

	
	
	
	

	18.
	a)
	Write a brief note on pushover analysis.




	[4M]

	
	b)
	Find the shape factor for a beam of circular section of radius R.
	[3M]

	
	c)
	Find the shape factor of a thin hollow circular cross-section.

	[3M]


-- 00 -- 00 –
H.T No





Regulations:


A15











PAGE  
Page 1 of 2

_1620720978

_1620720979

