[image: image3.jpg]Sreenidhi Institute of Science and Tm:hnnlngy

AUTONOMOUS







Sreenidhi Institute of Science and Technology

(An Autonomous Institution)
Code No: 7C354     






         Date: 20-Dec-2019 (FN)
B.Tech II-Year I-Semester External Examination, Dec-2019/Jan-2020 (Regular)

ANALOG ELECTRONIC CIRCUITS (CSE)
Time:
3 Hours







                    Max.Marks:70

 
Note:    a)  No additional answer sheets will be provided.

          

b)  All sub-parts of a question must be answered at one place only, otherwise it will not be valued.

          

c)  Missing data can be assumed suitably.








Part - A 



Max.Marks:20
Answer all QUESTIONS.
	1.
	Draw the V-I Characteristics of Zener diode and show the steps for calculation of  static and dynamic resistances.  
	[2M]

	2.
	Explain the transistor current components.
	[2M]

	3.
	Draw the frequency response of RC coupled amplifier and define bandwidth.
	[2M]

	4.
	In JFET, show that µ = gm rd.
	[2M]

	5.
	Explain any two biasing techniques of JFET.
	[2M]

	6.
	State Barkhausen criterion for oscillations.
	[2M]

	7.
	List the applications of PN Diode.
	[2M]

	8.
	Write the applications of MOSFET.
	[2M]

	9.
	Compare RC and LC oscillators.
	[2M]

	10.
	Compare BJT with FET.
	[2M]








       Part – B



   
 Max.Marks:50
ANSWER ANY FIVE QUESTIONS. EACH QUESTION CARRIES 10 MARKS.
	11.
	a)
	Derive the equation for diode current.
	[6M]

	
	b)
	A Germanium diode displays a forward voltage of 0.25V at 10mA at room temperature. Estimate the reverse saturation current.

	[4M]

	
	
	
	

	12.
	a)
	Derive the expression for Q-point coordinates for a CE configuration using collector base bias with emitter resistance. 
	[5M]

	
	b)
	Find the quiescent point for the circuit shown in the below figure.
  

[image: image1.png]vees





	[5M]

	
	
	
	

	13.
	a)
	Derive expressions for current gain, input resistance and voltage gain of Common Emitter amplifier using small signal approximate h-parameter model.
	[5M]

	
	b)
	Explain the operation of RC coupled amplifier with neat circuit diagram.
	[5M]

	
	
	
	

	14.
	a)
	Explain the operation of MOSFET  in enhancement and depletion modes.
	[3M]

	
	b)
	Explain the construction and operation of n-channel JFET with its drain  and transfer characteristics.
	[7M]

	
	
	
	

	15.
	a)
	Calculate the voltage gain and input impedance of given common source amplifier shown in  below figure with gm = 2mA/V, rd = 50KΩ, R1 = 10KΩ, R2 = 100MΩ,            Rs = 2KΩ and RD  = 5KΩ.       
         

[image: image2.emf]R1

VDD

RD

RS

2

R2

3

VDD

0

1



	[6M]

	
	b)
	Analyze CS amplifier with Voltage divider bias and find the operating point.
	[4M]

	
	
	
	

	16.
	a)
	Derive an expression for frequency of oscillations of Hartley Oscillator. 


	[5M]

	
	b)
	Find the frequency of oscillations of a transistorized Hartley Oscillator having L1=5mH, L2=5mH and C=0.01µF.
	[5M]

	
	
	
	

	17.
	a)
	Explain the operation of half wave rectifier with necessary waveforms. Derive VDC, IDC, ripple factor and efficiency for half wave rectifier.

	[4M]

	
	b)
	What is thermal runaway and how it can be avoided?
	[4M]

	
	c)
	Compare CE,CB and CC amplifiers.
	[2M]

	
	
	
	

	18.
	a)
	Explain why FET is called voltage controlled device.
	[4M]

	
	b)
	What are the advantages of FET over MOSFET?
	[4M]

	
	c)
	Explain the significance of Crystal oscillator.
	[2M]


-- 00 -- 00 –
H.T No





Regulations:


A18





SET-1





Vi








V0








PAGE  
Page 1 of 2

