[image: image1.jpg]Sreenidhi Institute of Science and Tm:hnnlngy

AUTONOMOUS






Sreenidhi Institute of Science and Technology

(An Autonomous Institution)
Code No: 5A405





                  
        Date: 28-Jan-2020 (FN)
B.Tech II-Year II-Semester External Examination, Jan/Feb - 2020 (Supplementary)

Electrical Machines – II (EEE)
Time:
3 Hours







                    Max.Marks:75

 
Note:    a)  No additional answer sheets will be provided.

          

b)  All sub-parts of a question must be answered at one place only, otherwise it will not be valued.

          

c)  Missing data can be assumed suitably.








Part - A 



Max.Marks:25
Answer all QUESTIONS.
	1.
	What is leakage flux in a single phase transformer?
	[3M]

	2.
	Define the voltage regulation of a transformer.
	[3M]

	3.
	What are the advantages of delta-delta connection?
	[3M]

	4.
	What is the difference between squirrel cage induction motor and slip ring induction            motor?
	[3M]

	5.
	Write the expression for the Maximum torque under running conditions?
	[3M]

	6.
	List out the different methods by which the speed control of Induction motor is     achieved.
	[2M]

	7.
	What are the conditions that must be satisfied in order to ensure that the transformers share the common load in proportion?
	[2M]

	8.
	What is the inter relation between the Rotor Input, Rotor copper loss and mechanical power developed?
	[2M]

	9.
	Draw the torque-slip characteristics of three phase Induction motor and explain the relation between torque and slip.
	[2M]

	10.
	How do you minimise the hysteresis and eddy current losses in single-phase transformers?
	[2M]









Part – B



   
 Max.Marks:50
ANSWER ANY FIVE QUESTIONS. EACH QUESTION CARRIES 10 MARKS.
	11.
	a)
	Explain the operation of single phase transformer on load with the help of phasor                                     diagrams
	[5M]

	
	b)
	The efficiency of a 20KVA, 2500/250V, single-phase transformer at unity p.f. is 98% at  rated load and also at half rated load. Determine 

(i) the transformer core loss and ohmic loss; and 

(ii) the p.u. values of the equivalent resistance of the transformer
	[5M]

	
	
	
	

	12.
	a)
	Discuss the parallel operation of single phase transformer with equal voltage ratios.
	[5M]

	
	b)
	Consider a 4KVA, 200/400V single phase transformers, supplying full load current at  

0.8 lagging power factor. The O.C/S.C test results are as follows:

O.C. test: 200V, 0.8A, 70W (L.V side)

S.C. test: 20V, 10A, 60W (H.V side)                                                                                

Calculate efficiency, secondary voltage and current into primary at the above load. 

Also calculate the load at unity power factor corresponding to maximum efficiency
	[5M]

	
	
	
	

	13.
	a)
	What are the different connections possible with delta-star combination and explain       with neat diagrams.  
	[5M]

	
	b)
	A ∆-∆ bank consisting of three 20KVA, 2300/230V transformer supplies a load of 

          40KVA. If one transformer is removed, find for the resulting V-V connection.         


(i) KVA load carried by each transformer.


(ii) percent of rated load carried by each transformer.


(iii) total KVA rating of the V-V bank.


(iv) ratio of the V-V bank to ∆-∆ bank transformer rating.


(v) percent increase in load on each transformer when bank is converter in         
      V-V bank
	[5M]

	
	
	
	

	14.
	a)
	Explain the production of Rotating magnetic field in three phase induction motors with   appropriate phasors.
	[5M]

	
	b)
	A 400V, 3-Φ, 50Hz, 4-pole, star-connected Induction Motor takes a line current of 10A with 0.86 p.f. lagging. Its total stator losses are 5% of the input. Rotor copper losses are 4% of the input to the rotor, and mechanical losses are 3% of the input to the rotor. Calculate (i) slip and rotor speed (ii) torque developed in rotor and (iii) shaft 

torque
	[5M]

	
	
	
	

	15.
	a)
	Draw the circle diagram for a 3.73KW, 200V, 50Hz, 4-pole, three-phase star connected Induction Motor from the following test data:

No-load: Line voltage 200V, line current 5A, total input 350W

Blocked rotor: Line voltage 100V, line current 26A, total input 1700W

Estimate the diagram for full load condition, the line current, power factor and also     

the maximum torque in terms of the full load torque. The rotor Cu loss at standstill is     

half the total Cu loss.
	[7M]

	
	b)
	Discuss the concepts of cogging and crawling in three phase Induction motors
	[3M]

	
	
	
	

	16.
	a)
	Explain the method of speed control by injecting emf into the rotor circuit.
	[5M]

	
	b)
	The full load efficiency and power factor of a 12 KW, 440V, 3-Φ Induction Motor are 

85% and 0.8 lag respectively. The blocked rotor line current is 45A at 220V. Calculate the ratio of starting to full load current, if the motor is provided with a star-delta starter. Neglect magnetising current.
	[5M]

	
	
	
	

	17.
	a)
	Discuss open circuit test on single phase transformer with a neat circuit diagram
	[4M]

	
	b)
	A 300KVA, 1- Φ transformer is designed to have a resistance of 1.5% and maximum   

efficiency occurs at a load of 173.2KVA. Find its efficiency when supplying full load at 

0.8 p.f lagging at normal voltage and frequency.
	[6M]

	
	
	
	

	18.
	a)
	Explain the construction of three-phase Induction Motor.
	[5M]

	
	b)
	A 12 pole, 3- Φ, 600V, 50Hz, star-connected Induction Motor has rotor resistance and standstill reactance of 0.03 and 0.5Ω/phase respectively. Calculate 
(i) speed at which maximum torque occurs. 
(ii) ratio of full-load torque to maximum torque if full-load speed is 495rpm.
	[5M]


-- 00 -- 00 –
H.T No





Regulations:


A15











PAGE  
Page 2 of 2

