[image: image5.jpg]Sreenidhi Institute of Science and Tm:hnnlngy

AUTONOMOUS






Sreenidhi Institute of Science and Technology

(An Autonomous Institution)
Code No: 5K502   





                  
        Date: 27-Dec-2019 (FN)
B.Tech III-Year I-Semester External Examination, Dec-2019/Jan-2020 (Supplementary)

Structural Analysis - II (CIVIL)
Time:
3 Hours







                    Max.Marks:75

 
Note:    a)  No additional answer sheets will be provided.

          

b)  All sub-parts of a question must be answered at one place only, otherwise it will not be valued.

          

c)  Missing data can be assumed suitably.








Part - A 



Max.Marks:25
Answer all QUESTIONS.
	1.
	Define ILD and its applications.
	[3M]

	2.
	List out the types of arches with neat sketches.
	[3M]

	3.
	Differentiate between two hinged and three hinged stiffening girders.
	[3M]

	4.
	List out the differences between stiffness and flexibility method. 
	[3M]

	5.
	What do you mean by sway in portal frames? Give two examples
	[3M]

	6.
	Define shape factor and load factor. 
	[2M]

	7.
	Draw ILD for reactions, SF for a simply supported beam with over hang.
	[2M]

	8.
	Analyse a propped cantilever beam subjected to UDL throughout its span adopt flexibility matrix method.

	[2M]

	9.
	Develop the stiffness matrix for a propped cantilever beam considering axial deformation. 
	[2M]

	10.
	Prove that arches produces lesser BM than a beam under same loading condition.
	[2M]










Part – B


   
 Max.Marks:50
ANSWER ANY FIVE QUESTIONS. EACH QUESTION CARRIES 10 MARKS.
	11.
	
	A series of loads 30 kN, 50 kN, 80 kN and 60 kN cross over a simply supported bridge of span 30m with the 30 kN load leading. Distance between them are 3.5 m (between 30 and 50 kNs), 3 m (between 50 and 80 kNs), and 2.5 m (between 80 and 60 kNs). Determine the position of 80kN load on bridge so that it creates maximum bending moment.
	[10M]

	
	
	
	

	12.
	
	Evaluate the expression for Horizontal Thrust ‘H’ and maximum B.M. for a two hinged Semi Circular arch of radius ‘R’ with eccentric point load.
	[10M]

	
	
	
	

	13.
	
	A cable is supported between two points 40m horizontally apart. The left support is 4m above the right support. The cable carries a load of 5kN/m on the horizontal span. The lowest point of the cable is 8m below the left support. Find the maximum tension in the cable.
	[10M]

	
	
	
	

	14.
	
	Analyze the continuous beam shown below by flexibility matrix method, if the downward settlement of supports B and C are 10mm and 5mm respectively. Take EI=18400 kN-m2                

                                                      [image: image1.jpg]


               
	[10M]

	
	
	
	

	15.
	
	Analyse the continuous beam shown below by using stiffness matrix method.

[image: image2.jpg]Yok 200k

Ml r
e





	[10M]

	
	
	
	

	16.
	
	Calculate the shape factor for of the I-section shown below. If the value of the yield stress is 250 N/mm2, find the plastic moment capacity of the section.

[image: image3.jpg]T

300mm

[50mm





	[10M]

	
	
	
	

	17.
	
	Generate the stiffness matrix for cantilever beam considering axial deformation.
	[10M]

	
	
	
	

	18.
	
	Calculate the plastic moment capacity and the collapse load for the beam shown below.

[image: image4.jpg]k?ﬂ*;m*m;l(zm%‘( 3m *_‘g{




	[10M]


-- 00 -- 00 –
H.T No





Regulations:


A15











PAGE  
Page 2 of 2

