[image: image1.jpg]Sreenidhi Institute of Science and Tm:hnnlngy

AUTONOMOUS






Sreenidhi Institute of Science and Technology

(An Autonomous Institution)
Code No: 5B626 





                  
      Date: 03-Feb-2020 (AN)
B.Tech III-Year II-Semester External Examination, Jan/Feb - 2020 (Supplementary)

Automobile Engineering (ME) 
Time:
3 Hours







                    Max.Marks:75

 
Note:    a)  No additional answer sheets will be provided.

          

b)  All sub-parts of a question must be answered at one place only, otherwise it will not be valued.

          

c)  Missing data can be assumed suitably.








Part - A 



Max.Marks:25
Answer all QUESTIONS.
	1.
	Define an Automobile.
	[3M]

	2.
	What is Carburetion?
	[3M]

	3.
	List the disadvantages of Air cooling system.
	[3M]

	4.
	What are various alternative fuels available for running an Automobile?
	[3M]

	5.
	Differentiate Hotch kiss drive and torque tube drive?
	[3M]

	6.
	Define camber.
	[2M]

	7.
	List various Lubricating oil filters used in Automobile.
	[2M]

	8.
	Describe the function of Condenser in Ignition system of an Automobile.
	[2M]

	9.
	Enumerate the requirements of Braking fluid.
	[2M]

	10.
	What are the Primary objects of Suspension system?
	[2M]


   


Part – B
 


   Max.Marks:50
ANSWER ANY FIVE QUESTIONS. EACH QUESTION CARRIES 10 MARKS.
	11.
	a)
	Discuss the advantages and disadvantages of rear wheel drive.
	[5M]

	
	b)
	Describe the transmission system used in the automobile system with neat sketch.
	[5M]

	
	
	
	

	12.
	a)
	Illustrate the operation of S.I Engine Mechanical Fuel pump.
	[5M]

	
	b)
	Explain the defects of simple carburettor .
	[5M]

	
	
	
	

	13.
	a)
	Explain the operation of forced circulation cooling system with neat sketch.
	[5M]

	
	b)
	Explain battery Ignition system with neat sketch.
	[5M]

	
	
	
	

	14.
	a)
	Give details of Bendix drive Mechanism.
	[5M]

	
	b)
	Explain Horn with neat sketch.
	[5M]

	
	
	
	

	15.
	a)
	Describe sliding Mesh Gear box with neat sketch.
	[5M]

	
	b)
	Paraphrase Double declutching .
	[5M]

	
	
	
	

	16.
	a)
	Explain Independent steering Linkage system.
	[5M]

	
	b)
	write any FOUR advantages of Independent suspension systems over rigid axle

suspension system.
	[5M]

	
	
	
	

	17.
	a)
	Explain Carter Carburetor with neat sketch
	[5M]

	
	b)
	Write the differentiate between splash and pressure lubrication system.
	[5M]

	
	
	
	

	18.
	a)
	Explain the operation of Cone clutch and write its advantages and disadvantages.
	[5M]

	
	b)
	Write the advantages of hydroulic blake sytsem.
	[5M]


-- 00 -- 00 –
H.T No





Regulations:


A15











PAGE  
Page 1 of 1

