[image: image4.jpg]Sreenidhi Institute of Science and Tm:hnnlngy

AUTONOMOUS






Sreenidhi Institute of Science and Technology

(An Autonomous Institution)
Code No: 4AC41         




                     
       Date: 24-Jan-2020 (FN)
B.Tech I-Year II-Semester External Examination, Jan/Feb-2020 (Supplementary)

BASIC ELECTRICAL ENGINEERING (BT)
Time:
3 Hours







                    Max.Marks:70

 
Note:    a)  No additional answer sheets will be provided.

          

b)  All sub-parts of a question must be answered at one place only, otherwise it will not be valued.

          

c)  Missing data can be assumed suitably.








Part - A 



Max.Marks:20
Answer all QUESTIONS.
	1.
	1. State Kirchhoff's laws.
	[2M]

	2.
	2. Define form factor and peak factor?
	[2M]

	3.
	3. What are the functions of yoke in DC machine?
	[2M]

	4.
	4. Give the emf equation of a transformer and define each term. 
	[2M]

	5.
	5. Why an induction motor never runs at its synchronous speed? 
	[2M]

	6.
	6. Write the advantages and disadvantages of PMMC instruments.
	[2M]

	7.
	Two resistances are connected in parallel with values 6 Ω and 3 Ω. This combination is connected in series with 8 Ω Find the equivalent resistance
	[2M]

	8.
	What do you mean by ideal transformer?
	[2M]

	9.
	What is the necessity of controlling torque in the instrument.
	[2M]

	10.
	Define slip of induction motor and give its formula.
	[2M]


                                                                           Part – B


   
 Max.Marks:50
ANSWER ANY FIVE QUESTIONS. EACH QUESTION CARRIES 10 MARKS.
	11.
	a)
	State and prove Maximum power transfer theorem.
	[5M]

	
	b)
	Using Thevenin’s theorem, find the current through R2 in the circuit shown in figure 1.  









[image: image1.png]ICPANEERTDL - " SETd-MicrosoftWord e | E | % |

=)

Home | Insert  Pagelmout  References  Maiings  Review  View ©
% cut T RIS
s Calbri Body) u -[A K 29| | nagbceoc| assbeeoe AaBbC aaBbce AAB aasbee. acsbeen aasben '%A & Repiace

paste — — o — . Emphasis - Change

" Frorf TR % @ < unitedTpar: T —— — N 3 == Emphasis <G ees | Ly Select~
Cipboar % editing
Home | View o =
oy Ij Xt | Hcop ‘/¢."\NOBDE,AWMW-‘ | =

N e | DR M ANoOOD g = Coray L

B S dhroate- || F A Q) B G A O Ayl & £ calors

| Ciipboard image Tools Shapes Cotors

- Ri=200 Re=300

B AN J J AN

B =L

- V=SV Re=1000 R=1200 ﬂf V20V

764, 213px 10 804 x 1220 w0 — O — @
°

Page:10f1 | Words: 35 znqum. fncia)

==





Figure 1


	[5M]

	
	
	
	

	12.
	
	Determine the form factor of the waveform  shown in figure 2.


           

[image: image2.png]



Figure 2


	[10M]

	
	
	
	

	13.
	a)
	Explain various losses that are occur in a DC machine and also give an expression for efficiency.
	[5M]

	
	b)
	An 8-pole lap-wound Dc generator armature has 960 conductors, a flux per pole of 40mwb and a speed of 400 r.p.m. Calculate the e.m.f generated on open circuit. If the same armature is wave wound, at what speed must it be driven to generate 400V?    
	[5M]

	
	
	
	

	14.
	a)
	Derive the EMF equation of a single phase transformer.
	[5M]

	
	b)
	Explain the principle of operation of transformer with neat diagram.
	[5M]

	
	
	
	

	15.
	a)
	Explain the principle of operation of three phase induction motor.
	[5M]

	
	b)
	A Three phase induction motor wound for 4 poles and is supplied from 50Hz system. Calculate i) synchronous speed 
ii) the speed of motor when slip is 4% 
iii) the rotor current frequency when the motor runs at 600 r.p.m.
	[5M]

	
	
	
	

	16.
	a)
	Explain the operation of Permanent Magnet Moving Coil (PMMC) instruments.
	[5M]

	
	b)
	A moving coil instrument whose resistance is 25Ω gives a full scale deflection with a current of 1mA. This instrument is to be used with a shunt resistance to extent its range to 100mA. Find the value of shunt resistance.
	[5M]

	
	
	
	

	17.
	a)
	Classify types of elements in the electrical circuits and explain them?
	[5M]

	
	b)
	Find current through the element AB for the circuit shown in figure 1 using nodal analysis? 

[image: image3.png]sa





	[5M]

	
	
	
	

	18.
	a)
	Explain about types of losses that occur in a transformer.
	[5M]

	
	b)
	Obtain the expression for torque of an induction motor.
	[5M]


-- 00 -- 00 –
H.T No





Regulations:


A14


























PAGE  
Page 2 of 2

