[image: image4.jpg]Sreenidhi Institute of Science and Tm:hnnlngy

AUTONOMOUS






Sreenidhi Institute of Science and Technology

(An Autonomous Institution)
Code No: 6A201    





                  
         Date: 03-Feb-2020 (FN)
B.Tech I-Year II-Semester External Examination, Jan/Feb-2020 (Supplementary)

CIRCUITS AND NETWORKS – I (EEE)
Time:
3 Hours







                    Max.Marks:75

 
Note:    a)  No additional answer sheets will be provided.

          

b)  All sub-parts of a question must be answered at one place only, otherwise it will not be valued.

          

c)  Missing data can be assumed suitably.








Part - A 



Max.Marks:25
Answer all QUESTIONS.
	1.
	What are the important features of inductance?
	[3M]

	2.
	What is a non-planar graph? Give an example.
	[3M]

	3.
	What are statically and dynamically induced emfs?
	[3M]

	4.
	Derive the expression for average value of a sinusoidal alternating quantity.
	[3M]

	5.
	Draw current locus diagram for series circuit with variable capacitance and constant resistance.
	[3M]

	6.
	State Compensation theorem.
	[2M]

	7.
	State and explain Kirchhoff’s current law.
	[2M]

	8.
	State the dot convention for coupled circuits.
	[2M]

	9.
	What are the conditions to be met for the application of reciprocity theorem?
	[2M]

	10.
	Draw the phasor diagrams for series RL circuit and series RC circuit.
	[2M]









 Part – B

   
 
Max.Marks:50
ANSWER ANY FIVE QUESTIONS. EACH QUESTION CARRIES 10 MARKS.
	11.
	a)
	Explain about ideal and practical sources with suitable diagrams and characteristics?
	[5M]

	
	b)
	Obtain the voltage drop across A-B terminals in the following circuit using Kirchhoff’s laws.
[image: image1.jpg]



	[5M]

	
	
	
	

	12.
	a)
	Explain the procedure for obtaining the fundamental cut-set matrix of a given network.
	[5M]

	
	b)
	Explain the procedure adopted for drawing the dual of a given network.
	[5M]

	
	
	
	

	13.
	a)
	Find the equivalent inductance of two coupled coils connected in parallel.
	[5M]

	
	b)
	A coil of 100 turns is wound uniformly over a insulator ring with a mean circumference of 2m and a uniform sectional area of 0.025 cm2. If the coil is carrying a current of 2A, then Calculate mmf of the circuit, magnetic field intensity and flux density.
	[5M]

	
	
	
	

	14.
	a)
	Explain about the response of pure capacitance to sinusoidal excitation.
	[5M]

	
	b)
	A 50Ω resistor is connected in parallel with an inductive reactance of 30Ω. A 20V signal is applied to the circuit. Find the total impedance and the line current in the circuit.
	[5M]

	
	
	
	

	15.
	a)
	Obtain the expression for the frequency at which maximum voltage across the capacitance in series resonance circuit in terms of Q-factor and resonance frequency.
	[5M]

	
	b)
	Determine the quality factor of a coil for the series circuit consisting of R=10Ω, L=0.1H and C=10μF.
	[5M]

	
	
	
	

	16.
	a)
	State and explain Reciprocity theorem.
	[5M]

	
	b)
	Determine the maximum power delivered to the load in the circuit shown in below figure.
[image: image2.jpg]\(b) 25 v 1051 Vo  FSL
— o W YWy

0 +
set -l
v 25 5. 50V
2] 105 28—
y + ES
2D /. X9 25 R
L
o 55
25 9_7
set 2
6P‘ 15 250
47

I(b)

seb 2





	[5M]

	
	
	
	

	17.
	a)
	Draw the oriented network graph from the incidence matrix given below.
Nodes

Branches

1

2

3

4

5

6

A

-1

0

0

1

-1

0

B

1

-1

0

0

0

-1

C

0

1

-1

0

1

0

D

0

0

1

-1

0

1


	[5M]


	
	b)
	Using nodal analysis, find the current through 5Ω resistor in the following circuit.
[image: image3.jpg]



	[5M]

	
	
	
	

	18.
	a)
	State and explain Thevenin’s theorem
	[5M]

	
	b)
	With respect to a series resonant circuit, prove that the bandwidth is inversely proportional to Q-factor at resonance.

	[5M]


-- 00 -- 00 –
H.T No





Regulations:


A17











PAGE  
Page 2 of 2

