[image: image10.jpg]Sreenidhi Institute of Science and Tm:hnnlngy

AUTONOMOUS






Sreenidhi Institute of Science and Technology

(An Autonomous Institution)
Code No: 4A304





                 
      Date: 03-Nov-2020 (FN)
B.Tech II-Year I-Semester External Examination, Oct/Nov - 2020 (Supplementary)
CIRCUITS & NETWORKS – II (EEE)

Time:
 2 Hours







                    Max.Marks:70


 
Note:    a)  No additional answer sheets will be provided.

          

b)  All sub-parts of a question must be answered at one place only, otherwise it will not be valued.

          

c)  Missing data can be assumed suitably.

ANSWER ANY 5 OUT OF 8 QUESTIONS. EACH QUESTION CARRIES 14 MARKS.

	1.
	a)
	Three equal impedances each of (8+j10)Ω, are connected in star. This is further connected to a 440V, 50Hz, 3-ϕ phase supply. Calculate 

(i) phase voltage   (ii) phase current     (iii) active power and      (iv) reactive power.
	[7M]

	
	b)
	A symmetrical 440V, 3-ϕ system supplied a star-connected load with the ZR=10Ω, ZY=j5Ω, ZB= -j5Ω as shown in below fig. Calculate line currents and voltage across each phase impedance by Millman’s theorem. The phase sequence is RYB.

[image: image1.png]



	[7M]

	
	
	
	

	2.
	a)
	In the network shown in below figure the switch is closed. Assuming all the initial condition as zero, find I, di/dt and d2i/dt2 at t=0+.

[image: image2.png]\0

wah

I’%]"NMH
N
pRIG)

lOMF




	[7M]

	
	b)
	In the network shown in below figure, the switch is closed at t=0. Find the currents i1(t) and i2(t) when initial current through the inductor is zero and initial voltage on the capacitor is 4v using Laplace transform.

[image: image3.png]



	[7M]

	
	
	
	

	3.
	a)
	Determine the driving point impedance of the network shown in below figure.

[image: image4.png]T(5)




	[7M]

	
	b)
	For the network shown in below figure, plot poles and zeros of function of I2/I1.

[image: image5.png]1a

v

— J50MF

(GRS

200N




	[7M]

	
	
	
	

	4.
	a)
	Find the open circuit impedance parameters for the network shown in below figure. Determine whether the network is symmetrical and reciprocal.

[image: image6.png]



	[7M]

	
	b)
	Find Y parameters for the network shown in below figure.

[image: image7.png]\&

—

+




	[7M]

	
	
	
	

	5.
	a)
	A Π-section filter network consists of a series arm inductor of 20mH and two shunt arm capacitors of 0.16µF each. Calculate the cut-off frequency, and attenuation and phase shift at 15kHz. What is the value of the nominal impedance in the pass band.
	[7M]

	
	b)
	In a constant-K band pass filter, the ratio of capacitances in the shunt and series arms is 100:1. The resonant frequency of both arms is 1000Hz. Find the bandwidth.
	[7M]

	
	
	
	

	6.
	a)
	Find the exponential Fourier series of the waveform shown in below figure. Draw the magnitude spectrum.

[image: image8.png]



	[7M]

	
	b)
	Find the Fourier transform of the function shown in below figure.

[image: image9.png]



	[7M]

	
	
	
	

	7.
	a)
	Obtain an expression for phase angle of star connected load by using two watt meter method and by using necessary phasor diagrams.
	[7M]

	
	b)
	Derive exponential response of series RC circuit.
	[7M]

	
	
	
	

	8.
	a)
	Derive the expressions and draw the equivalent circuit for representing a two port network by means of Hybrid parameters.
	[7M]

	
	b)
	Write about analysis of Band stop filter.
	[7M]


-- 00 -- 00 –
H.T No





Regulations:


A14











PAGE  
Page 1 of 2

