[image: image2.jpg]Sreenidhi Institute of Science and Tm:hnnlngy

AUTONOMOUS






Sreenidhi Institute of Science and Technology

(An Autonomous Institution)
Code No: 5B730  





                 
      Date: 06-Oct-2020 (AN)
B.Tech IV-Year I-Semester External Examination, October - 2020 (Supplementary)
Finite Element Method (ME)
Time:
 2 Hours







                    Max.Marks:75


 
Note:    a)  No additional answer sheets will be provided.

          

b)  All sub-parts of a question must be answered at one place only, otherwise it will not be valued.

          

c)  Missing data can be assumed suitably.

ANSWER ANY FIVE QUESTIONS. EACH QUESTION CARRIES 15 MARKS.
	1.
	a)
	Derive the element stiffness matrix for a 1-D bar element. State the properties of stiffness matrix.
	[8M]

	
	b)
	For a stepped bar shown in figure.1 using elimination method of handling boundary conditions. Determine the following.

i) Nodal displacements  ii) Stress in each element. Take E= 200GPa.
                            150mm2                  240mm2
                           

                    75mm     75mm      60mm           

Figure.1
	[7M]

	
	
	
	

	2.
	a)
	What is a beam? Derive the hermite shape functions for beam element.
	[8M]

	
	b)
	Find the nodal displacement, stress and reactions of truss element shown in figure.2. Take E=200GPa   Area=200mm2.
                                                              12kN     

 

                   300mm    

    - - - --  - - - - - - - - - - - -  -   -      

       500mm           400mm        

Figure.2
	[7M]

	
	
	
	

	3.
	a)
	Define Iso-parametric, Super-parametric and Sub-parametric elements.
	[8M]

	
	b)
	An axisymmetric ring element is shown in figure.3. Derive the strain displacement Matrix. Take E=2x105N/mm2.                                                                                                  

                                                                  2

                                                                        


                                               50mm


  .                                                           1               3        

                                                            100mm 50mm

                                                    

Figure. 3.


	[7M]

	
	
	
	

	4.
	
	An isoparametric element is shown in figure. 4(a), and a corresponding isoparametric distorted element is shown in figure. 4(b).Discuss the transformation that relates partial derivatives in the original x, y coordinates to the generalized ξ, η coordinates.

[image: image1.jpg]n=-1 —
(a) Parent element (b) Distorted element




Figure. 4


	[15M]

	
	
	
	

	5.
	a)
	Derive the element conductivity matrix for solving 1-D heat conduction problems.
	[8M]

	
	b)
	A composite slab consists of 3 materials of different  conductivities
i.e. 22 W/m K, 32 W/m K, 52 W/m K of thickness 0.31m, 0.14m and

0.14m respectively. The outer surface temperature is 22 ºC and the

inner surface is exposed to the convective heat transfer coefficient

of 28 W/m2 K, at 800 ºC. Determine the temperature distribution

within the wall.
	[7M]

	
	
	
	

	6.
	
	Determine the eigen values of the stepped bar shown in figure.5. 

Take E=200GPa, weight density 7850 kg/m3.
                                     A1= 1200mm2    A2= 900mm2

                                                                                    x            

                             300mm                 400mm

Figure. 5.
	[15M]

	
	
	
	

	7.
	a)
	Discuss the Engineering applications of FEM.
	[8M]

	
	b)
	Differentiate between local and global coordinates? Give examples.
	[7M]

	
	
	
	

	8.
	a)
	What is a higher order element? List some advantages of using 3-D elements in structural mechanics.
	[8M]

	
	b)
	Name a few boundary conditions involved in any heat transfer analysis. 
	[7M]


-- 00 -- 00 –
H.T No





Regulations:


A15








20kN








20Kn











   10kN








PAGE  
Page 1 of 2

