[image: image1.png]1000N 1000





Sreenidhi Institute of Science and Technology

(An Autonomous Institution)
Code No: 7K201





                 
      Date: 23-Nov-2020 (FN)
B.Tech I-Year II-Semester External Examination, Nov/Dec - 2020 (Regular & Supplementary)
ENGINEERING MECHANICS FOR CIVIL ENGINEERING (CIVIL)
Time:
 2 Hours







                    Max.Marks:70


 
Note:    a)  No additional answer sheets will be provided.

          

b)  All sub-parts of a question must be answered at one place only, otherwise it will not be valued.

          

c)  Missing data can be assumed suitably.

ANSWER ANY 5 OUT OF 8 QUESTIONS. EACH QUESTION CARRIES 14 MARKS.

	1.
	a)
	Two equal weights each of 1000N is supported by a flexible string as shown in fig below. Find the tensions in the portion AB and CD of the string.

[image: image7.jpg]Sreenidhi Institute of Science and Tm:hnnlngy

AUTONOMOUS





	[7M]

	
	b)
	Two forces P and Q are applied as shown in Fig. 1, to an aircraft connection. Knowing that the connection is in equilibrium and that P = 500 N and Q = 650 N, determine the magnitudes of the forces exerted on the rods A and B.

[image: image2.jpg]



	[7M]

	
	
	
	

	2.
	
	A uniform ladder AB of length l=20 m and weight W is supported by the horizontal floor at A and by a vertical wall at B. It makes an angle 450 with the horizontal. If a man, whose weight is one half that of the ladder, ascends the ladder, how much length x of the ladder he shall climb before the ladder slips. If a boy now stands on the end A, what must be his least weight w so that the man may go on the top of the ladder? Assume,µ between the ladder and the wall=1/3, µ between the ladder and the floor=1/2
	[14M]

	
	
	
	

	3.
	
	An I-section is made up of three rectangles as below Figure.  Find the moment of inertia of the section. 


[image: image3.png]



	[14M]

	
	
	
	

	4.
	
	Determine the Horizontal and vertical components of reactions at A and B of frame which made of three squares and three hinged as shown in below figure using virtual energy method.

[image: image4.jpg]P=1000N Q=1000N





	[14M]

	
	
	
	

	5.
	
	A Projectile is fired with velocity of 500m/s at an inclination of 300

i.  Find the velocity and Direction of Projectile after 30sec of travel.

ii. Determine the time of flight.

iii. Find out maximum height of projectile. 
	[14M]

	
	
	
	

	6.
	a)
	A body moves along a straight line so that its displacement from a fixed point on the line is given by s = 3t2 + 2t. Find the displacement, velocity and acceleration at the end of t = 3s.
	[7M]

	
	b)
	An open platform elevator is moving down a mine shaft with an acceleration 0.5 m/s2. After the elevator has travelled 25m (from the top shaft) a stone dropped from the shaft determine a) the time taken the stone to hit the elevator the distance travelled by the elevator at the time of impact.  
	[7M]

	
	
	
	

	7.
	
	Three cylindersare piled up  in a rectangular channel as shown in Figure below. Determine the reaction R6 between the cyclinder A and the vertical wall of the channel.

[image: image5.jpg]



	[14M]

	
	
	
	

	8.
	
	A horizontal bar 1 m long and of negligible weight rests on rough inclined planes shown in below fig. If the angle of friction is 150, determine the minimum value of x at which the load Q = 200N may be applied before slipping impends.

[image: image6.jpg]



	[14M]


-- 00 -- 00 –
H.T No





Regulations:


A18











PAGE  
Page 1 of 2

_1667626677

