H.T No

Regulations:

A17

Sreenidhi Institute of Science and Technology

(An Autonomous Institution)

Code No: 6C509   						                 	      Date: 13-Mar-2021(FN)

B.Tech III-Year I- Semester External Examination, March/April - 2021 (Supplementary)

DIGITAL SIGNAL PROCESSING (ECE)

Time:	 3 Hours								                    Max.Marks:75

Note:    a)  No additional answer sheets will be provided.

b)  All sub-parts of a question must be answered at one place only, otherwise it will not be valued.

c)  Missing data can be assumed suitably.

ANSWER ANY 5 OUT OF 8 QUESTIONS. EACH QUESTION CARRIES 15 MARKS.

Bloom's Cognitive Levels of Learning (BCLL)

| Remember | L1 | Apply | L3 | Evaluate | L5 |
| --- | --- | --- | --- | --- | --- |
| Understand | L2 | Analyze | L4 | Create | L6 |

|  |  |  | BCLL | CO(s) | Marks |
| --- | --- | --- | --- | --- | --- |
| 1. | a) | Check whether the following systems are static or dynamic, linear or nonlinear, time variant or invariant, causal or noncausal, stable or unstable. (i) y[𝒏] = 𝐜 𝒙(𝒏) (ii) y[𝒏] = 𝒙(−𝒏 + 𝟐) (iii) y[𝒏] = 𝒙(𝟐𝒏) (iv) y[𝒏] = 𝒙(𝒏) 𝐜𝐨s(ω𝟎𝒏) | L3 | CO1 | [8M] |
|  | b) | Determine the frequency response for the system given by y[n]-3/4y[n-1]+1/8 y[n-2] = x[n]- x[n-1] | L5 | CO1 | [7M] |
| 2. | a) | State and prove the periodicity and time reversal properties of DFT. | L5 | CO2 | [8M] |
|  | b) | Compute the linear convolution of the sequences x1[n] = {1, 2, 3} and x2[n] = {4, 5, 6, 7, 8} through circular convolution. | L3 | CO2 | [7M] |
| 3. | a) | Obtain the 8-point DFT using DIT-FFT algorithm for x[n]={1, 1, 1, 1, 1, 1, 1, 1}. | L4 | CO3 | [8M] |
|  | b) | Compute the DFT for the sequence (0.5,0.5,0.5,0.5,1,1,1,1) using DIF-FFT. | L3 | CO3 | [7M] |
| 4. | a) | Develop cascade and parallel realization of the system given by the difference equation y[n]+(3/8) y[n-1]–(3/32)y[n-2]–(1/64)y[n-3]=x[n]+3x[n-1]+2x[n-2]. | L4 | CO4 | [8M] |
|  | b) | Find the Z-transform of x[n]=(1/3)sin(nπ/4) u[n] | L3 | CO4 | [7M] |
| 5. | a) | Discuss the steps in the design of IIR filter using bilinear transformation for any one type of filter. | L2 | CO5 | [8M] |
|  | b) | Design a linear phase FIR low pass filter using rectangular window by taking 7 samples of window sequence with a cut off frequency of 0.2πrad/sec. also plot the frequency response. | L4 | CO5 | [7M] |
| 6. | a) | Derive and explain the operation of sampling rate conversion by a non-integer. | L5 | CO6 | [8M] |
|  | b) | Discuss the applications of multi-rate signal processing. | L2 | CO6 | [7M] |
| 7. | a) | Discuss the concept of stability and causality with examples. | L2 | CO1 | [5M] |
|  | b) | Compute the circular convolution of x1[n] = {1, 2, 3, 4} and x2[n] = {5, 6, 7, 8}. | L3 | CO2 | [5M] |
|  | c) | Find the IDFT of X[k]={1, 1, 2, 5}. | L6 | CO3 | [5M] |
| 8. | a) | Obtain the direct form II and cascade Realization for the system y[n] = −0.1 y[n −1] + 0.2 y[n − 2] + 3x[n] + 3.6x[n −1] + 0.6 x[n –2] | L4 | CO4 | [5M] |
|  | b) | Compare IIR and FIR filters. | L5 | CO5 | [5M] |
|  | c) | What do you mean by aliasing? Explain. | L3 | CO6 | [5M] |

-- 00 -- 00 –