[image: image1.jpg]Sreenidhi Institute of Science and Tm:hnnlngy

AUTONOMOUS






Sreenidhi Institute of Science and Technology

(An Autonomous Institution)
Code No: 6B515  





                 
      Date: 05-Oct-2021(FN)
B.Tech III-Year I- Semester External Examination, Sept/Oct - 2021 (Supplementary)
METAL CUTTING AND MACHINE TOOLS (ME)
Time:
 3 Hours







                    Max.Marks:75


 
Note:    a)  No additional answer sheets will be provided.

          

b)  All sub-parts of a question must be answered at one place only, otherwise it will not be valued.

          

c)  Missing data can be assumed suitably.

ANSWER ANY 5 OUT OF 8 QUESTIONS. EACH QUESTION CARRIES 15 MARKS.

Bloom's Cognitive Levels of Learning (BCLL)

	Remember
	L1
	Apply
	L3
	Evaluate
	L5

	Understand
	L2
	Analyze
	L4
	Create
	L6


	
	 
	
	BCLL
	CO(s)
	Marks

	1.
	a)
	Draw the Merchant’s circle diagram and derive expressions to show relationships among the different forces?
	L1
	CO1
	[8M]

	
	b)
	In an orthogonal cutting operation on 60mm dia work the following data have been observed:

Cutting Speed =100m/min; Uncut chip thickness=0.06 mm

Width of Cut=3.83 mm; Chip Thickness Ratio=0.51; Rake angle=20°

Cutting force=363 N; Thrust force=127 N

Determine shear angle, friction angle, shear stress along the shear plane, chip velocity, shear strain in chip, and the power for the cutting operation.
	L5
	CO1
	[7M]

	
	
	
	
	
	

	2.
	a)
	What are crater wear and flank wear? Explain.
	L2
	CO2
	[8M]

	
	b)
	Derive the expression for Optimum cutting speed under minimum cost criteria.
	L4
	CO2
	[7M]

	
	
	
	
	
	

	3.
	
	Draw a neat Sketch of a shaper. Explain the parts of a shaper Briefly.
	L1
	CO3
	[15M]

	
	
	
	
	
	

	4.
	
	Sketch and describe the following milling operations: 

i)Straddle Milling 

 ii)Gang Milling  

iii) Form Milling 

iv) End milling  

v) Milling T slots
	L2
	CO4
	[15M]

	
	
	
	
	
	

	5.
	a)
	Explain the terms ‘Loading’ and ‘Glazing’ of grinding wheel.
	L2
	CO5
	[8M]

	
	b)
	What is Honing? Explain the Vertical Honing machine with sketch.
	L1
	CO5
	[7M]

	
	
	
	
	
	

	6.
	a)
	Explain the 3-2-1 location principle used in design of JIgs and Fixtures.
	L2
	CO6
	[8M]

	
	b)
	Explain Abrasive Jet machining (AJM) Briefly.
	L2
	CO6
	[7M]

	
	
	
	
	
	

	7.
	a)
	Derive the expressions to find out shear velocity and chip flow velocity when cutting velocity, shear angle  and rake angle are known.
	L3
	CO1
	[5M]

	
	b)
	What are the different sources of heat generation in metal cutting? Explain.
	L2
	CO2
	[5M]

	
	c)
	 What is Taper turning? Explain any one  taper turning methods.
	L2
	CO3
	[5M]

	
	
	
	
	
	

	8.
	a)
	What is indexing? Explain any one indexing method.
	L1
	CO4
	[5M]

	
	b)
	Explain the following 
i) Dressing    

ii)Truing
	L2
	CO5
	[5M]

	
	c)
	What are the applications of Electro Discharge Machining?
	L2
	CO6
	[5M]


-- 00 -- 00 –
H.T No





Regulations:


A17











PAGE  
Page 1 of 1

