[image: image2.jpg]Sreenidhi Institute of Science and Tm:hnnlngy

AUTONOMOUS






Sreenidhi Institute of Science and Technology

(An Autonomous Institution)
Code No: 6C727   





                 
      Date: 01-Sept-2021 (FN)
B.Tech IV-Year I- Semester External Examination, Aug/Sept - 2021 (Supplementary)
DIGITAL DESIGN THROUGH VERILOG (ECE)
Time:
 3 Hours







                    Max.Marks:75


 
Note:    a)  No additional answer sheets will be provided.

          

b)  All sub-parts of a question must be answered at one place only, otherwise it will not be valued.

          

c)  Missing data can be assumed suitably.

ANSWER ANY 5 OUT OF 8 QUESTIONS. EACH QUESTION CARRIES 15 MARKS.

Bloom's Cognitive Levels of Learning (BCLL)

	Remember
	L1
	Apply
	L3
	Evaluate
	L5

	Understand
	L2
	Analyze
	L4
	Create
	L6


	
	 
	
	BCLL
	CO(s)
	Marks

	1.
	a)
	Explain the ASIC design flow briefly.
	L2
	CO1
	[8M]

	
	b)
	Explain the activities involved in Physical Design Process.
	L2
	CO1
	[7M]

	
	
	
	
	
	

	2.
	a)
	Explain the blocking and non-blocking statements in Verilog with suitable example.
	L2
	CO2
	[8M]

	
	b)
	Write the 2:4 Priority Encoder Verilog HDL code.
	L1
	CO2
	[7M]

	
	
	
	
	
	

	3.
	a)
	Design a 4-bit parallel adder and write a test bench to verify its functionality.
	L6
	CO3
	[8M]

	
	b)
	Describe Gate Delays. Write the Verilog HDL code for following schematic diagram and draw the timing diagram for following two cases.

Case 1: Assume any one of the input is “0” of “g2” instance and assume input “c1” is always “High”  

Case 2: Assume any one of the input is “0” of “g1” instance and assume input “c2” is always “High”

[image: image1.png]., Digital Design through verilog HDL By Padma Nabhan, Bala Tripura
5

FEile Edit View Wi Help
ARZRBEOEE| ®®[w/a|=@® == | HB|S z | Tools | Sign | Comment
= A
98 GATE LEVEL MODELING -2
L)
N b1 e—] p
gt
cle at
a
b2 e a—s
o2 hod S
c2 o— R

Figure 5.28(b) The circui

<3 % B





Assume 


i
Rise Time Delay is “2” and Fall Time Delay is “1” for wires a1,a2;

ii
Rise Time Delay is “3” and Fall Time Delay is “4” for g1;

iii
Rise Time Delay is “5” and Fall Time Delay is “6” for g2;

iv
Rise Time Delay is “8” and Fall Time Delay is “7” for g3;
	L1
	CO3
	[7M]

	
	
	
	
	
	

	4.
	a)
	Apply User Defined Primitive for JK Flip Flop.
	L3
	CO4
	[8M]

	
	b)
	What is contention resolution and explain how it can be eliminated with suitable example.
	L2
	CO4
	[7M]

	
	
	
	
	
	

	5.
	a)
	Describe combinational testing with examples.
	L1
	CO5
	[8M]

	
	b)
	Explain the assert-one-hot monitor with the help of an example. 
	L2
	CO5
	[7M]

	
	
	
	
	
	

	6.
	
	Design a vending machine in Verilog HDL.
	L6
	CO6
	[15M]

	
	
	
	
	
	

	7.
	a)
	Explain the terms Scalars and Vectors.
	L2
	CO1
	[5M]

	
	b)
	Write Verilog HDL source code for a data level description of 4 to 1 multiplexer circuit.
	L1
	CO2
	[5M]

	
	c)
	Discuss the Time Delays associated with gates.
	L2
	CO3
	[5M]

	
	
	
	
	
	

	8.
	a)
	Distinguish Tasks and Function with examples.
	L2
	CO4
	[5M]

	
	b)
	Describe test bench and mention few  techniques.
	L1
	CO5
	[5M]

	
	c)
	Illustrate the ASM chart for Dice game controller.  
	L4
	CO6
	[5M]


-- 00 -- 00 –
H.T No





Regulations:


A17











PAGE  
Page 1 of 1

