[image: image1.jpg]Sreenidhi Institute of Science and Tm:hnnlngy

AUTONOMOUS






Sreenidhi Institute of Science and Technology

(An Autonomous Institution)
Code No: 5B101 





                 
      Date: 03-Nov-2021 (FN)
B.Tech I-Year I- Semester External Examination, Oct/Nov-2021 (Supplementary)
ENGINEERING DRAWING - I (Common to All)
Time:
 3 Hours







                    Max.Marks:75


 
Note:    a)  No additional answer sheets will be provided.

          

b)  All sub-parts of a question must be answered at one place only, otherwise it will not be valued.

          

c)  Missing data can be assumed suitably.

ANSWER ANY 5 OUT OF 8 QUESTIONS. EACH QUESTION CARRIES 15 MARKS.

Bloom's Cognitive Levels of Learning (BCLL)

	Remember
	L1
	Apply
	L3
	Evaluate
	L5

	Understand
	L2
	Analyze
	L4
	Create
	L6


	
	 
	
	BCLL
	CO(s)
	Marks

	1.
	
	The vertex of a hyperbola is 65 mm from its focus. Draw the curve if the eccentricity is 3/2 · Draw a normal and a tangent at a point on the curve, 75 mm from the directrix. 
	L4
	CO1
	[15M]

	
	
	
	
	
	

	2.
	
	A line PQ 100 mm long, is inclined at 30° to the H.P. and at 45° to the V.P. Its mid-point is in the V.P. and 20 mm above the H.P. Draw its projections, if its end P is in the third quadrant and Q in the first quadrant.
	L5
	CO2
	[15M]

	
	
	
	
	
	

	3.
	
	A square ABCO of 50 mm side has its corner A in the H.P., its diagonal AC inclined at 30° to the H.P. and the diagonal BO inclined at 45° to the V.P. and parallel to the H.P. Draw its projections.
	L4
	CO3
	[15M]

	
	
	
	
	
	

	4.
	
	A pentagonal pyramid, base 25 mm side and axis 50 mm long has one of its triangular faces in the V.P. and the edge of the base contained by that face makes an angle of 30° with the H.P. Draw its projections.
	L5
	CO4
	[15M]

	
	
	
	
	
	

	5.
	
	A triangular pyramid, having base 40 mm side and axis 50 mm long, is lying on the H.P. on one of its faces, with the axis parallel to the V.P. A section parallel to the V.P. cuts the pyramid at a distance of 6 mm from the axis. Draw its sectional front view and the top view.
	L4
	CO5
	[15M]

	
	
	
	
	
	

	6.
	
	A vertical square prism, base 50 mm side and height 90 mm has a face inclined at 30° to the V.P. It is completely penetrated by another square prism, base 38 mm side and axis 100 mm long, faces of which are equally inclined to the V.P. The axes of the two prisms are parallel to the V.P. and bisect each other at right angles. Draw the projections showing lines of intersection.
	L5
	CO6
	[15M]

	
	
	
	
	
	

	7.
	
	A line PQ 75 mm long, has its end P in the V.P. and the end Q in the H.P. The line is inclined at 30° to the H.P. and at 60° to the V.P. Draw its projections.
	L3
	CO1
	[15M]

	
	
	
	
	
	

	8.
	
	A square prism, base 40 mm side and height 65 mm, has its axis inclined at 45° to the H.P. and has an edge of its base, on the H.P and inclined at 30° to the V.P. Draw its projections.

	L4
	CO4
	[15M]


-- 00 -- 00 –
H.T No





Regulations:


A15











PAGE  
Page 1 of 1

