[image: image7.jpg]Sreenidhi Institute of Science and Tm:hnnlngy

AUTONOMOUS






Sreenidhi Institute of Science and Technology

(An Autonomous Institution)
Code No: 8AC42    





                 
      Date: 05-Nov-2021 (FN)
B.Tech I-Year I- Semester External Examination, Oct/Nov-2021 (Supplementary)
ELECTRICAL CIRCUITS AND NETWORK ANALYSIS (ECM)
Time:
 3 Hours







                    Max.Marks:70


 
Note:    a)  No additional answer sheets will be provided.

          

b)  All sub-parts of a question must be answered at one place only, otherwise it will not be valued.

          

c)  Missing data can be assumed suitably.

ANSWER ANY 5 OUT OF 8 QUESTIONS. EACH QUESTION CARRIES 14 MARKS.

Bloom's Cognitive Levels of Learning (BCLL)

	Remember
	L1
	Apply
	L3
	Evaluate
	L5

	Understand
	L2
	Analyze
	L4
	Create
	L6


	
	 
	
	BCLL
	CO(s)
	Marks

	1.
	a)
	In the circuit shown below, determine the current through 2Ω resistor and the total current delivered by the battery 10V. Use Kirchhoff’s laws. 


[image: image1.png]a0 10

50 EXeS





	L4
	CO1
	[7M]

	
	b)
	Find the current in the 4Ω resistor using nodal analysis.


	L4
	CO1
	[7M]

	
	
	
	
	
	

	2.
	a)
	In a series circuit containing a pure resistance and a pure inductance, the current and voltage are expressed as i(t) = 5 sin (314t + 2π/3) and V(t) = 15 sin (314t+5π/6). Find the impedance of the circuit, resistance and inductance of the circuit
	L4
	CO2
	[7M]

	
	b)
	A Series R-L-C Circuit is resonant at 1 megacycles/sec. Its bandwidth is 5000 cycles per second and input impedance at resonance is 50Ω. Find R, L and C.
	L3
	CO2
	[7M]

	
	
	
	
	
	

	3.
	a)
	In mutually coupled coils, the voltage drop across one coil, from the dotted terminal to the undotted terminal, equals M times the derivative of the current through the other coil, from the dotted terminal to the undotted terminal. Explain.
	L5
	CO3
	[7M]

	
	b)
	A coil having an inductance of 100mH is magnetically coupled to another coil having an inductance of 900mH. The coefficient of coupling between the coils is 0.45. Calculate the equivalent inductance if the two coils are connected in i) series aiding  
ii) series opposing iii) parallel aiding and iv) parallel opposing.
	L3
	CO3
	[7M]

	
	
	
	
	
	

	4.
	a)
	Find the Norton’s equivalent circuit across a-b for the network shown in below figure.
[image: image2.png]@ - | @ Search the web
M Queston Paper setting for SNIST X | @ NA 176C35 QUESTION BANK (€ X | @ KTU Universty Question Bank - 16 KTUnebpdt X |+

C @ File | C/Users/peddachenchi%20reddy/Downloads/EE201-E_S3Dec18_KTUweb.pdf

EE201-E_S3Dec18_KTUweb.pdf

Page10f3

Fig2

10 a) Find current, "’ in the network shown in Fig.3 using super position theorem

b i

"3 9 IJE

©)





	L4
	CO4
	[7M]

	
	b)
	Find the current i in the network shown. below using Superposition theorem

[image: image3.png]@~ | 2 search the web... maen-0:

C @ File | Dy/Question%20papers/SNSIT/EE201-E_S3Dec18_KTUweb%20(1).pdf % = E 3

B R3913 Pages: 3

Fig2
10 a) Find current, 7’ in the network shown in Fig3 using super position theorem ©)
A —2 !
10 5Q
3% CP 1A iy
Fig3
b) List the properties of Incidence Matrix “)

11 For the network shown in Fig4 write down the tieset matrix and obtain the  (10)
network equilibrium equations in matrix form using KVL. Calculate the loop
currents.

534 AM
5/03/2021





	L4
	CO4
	[7M]

	
	
	
	
	
	

	5.
	a)
	Find the transmission parameters of the below network

[image: image4.png]-





	L4
	CO5
	[7M]

	
	b)
	What are h- parameters? Draw the equivalent circuit of a two port network with h- parameter representation.
	L2
	CO5
	[7M]

	
	
	
	
	
	

	6.
	a)
	Find the response of RC network with AC excitation using Laplace transform method.
	L2
	CO6
	[7M]

	
	b)
	The circuit shown in figure consists of resistance, inductance, and capacitance in series with a 200 V constant source. When the switch is closed at t = 0. Find the transient current using differential equation approach.
[image: image5.emf]
	L3
	CO6
	[7M]

	
	
	
	
	
	

	7.
	a)
	An ideal voltage source whose internal resistance rs=0 cannot be transformed into a current source. Explain.
	L5
	CO1
	[5M]

	
	b)
	Find the effective value of the periodic voltage waveform shown in the below figure
[image: image6.png]50V




	L3
	CO2
	[5M]

	
	c)
	Find the equivalent reluctance of a closed magnetic circuit consisting of three magnetic cores.
	L3
	CO3
	[4M]

	
	
	
	
	
	

	8.
	a)
	State and prove the Maximum Power Transfer Theorem when the load resistance is equal to the magnitude of source impedance.
	L2
	CO4
	[5M]

	
	b)
	Show that the overall admittance parameter matrix for parallel connected two port network is the sum of admittance parameters of each individual two port network in parallel.
	L3
	CO5
	[5M]

	
	c)
	Briefly explain the importance of initial conditions.
	L2
	CO6
	[4M]


-- 00 -- 00 –
H.T No





Regulations:


A20





6Ω





10V





5Ω





3Ω





4Ω





5V





2Ω














PAGE  
Page 2 of 2

_1676369096

_1676335885

