H.T No

Regulations:

A15

Sreenidhi Institute of Science and Technology

(An Autonomous Institution)

Code No: 5C405    						                 	      Date: 13-July-2021 (AN)

B.Tech II-Year II- Semester Pending External Examination, July - 2021 (Supplementary)

ANALOG COMMUNICATIONS (ECE)

Time:	 3 Hours								                    Max.Marks:75

Note:    a)  No additional answer sheets will be provided.

b)  All sub-parts of a question must be answered at one place only, otherwise it will not be valued.

c)  Missing data can be assumed suitably.

ANSWER ANY 5 OUT OF 8 QUESTIONS. EACH QUESTION CARRIES 15 MARKS.

Bloom's Cognitive Levels of Learning (BCLL)

| Remember | L1 | Apply | L3 | Evaluate | L5 |
| --- | --- | --- | --- | --- | --- |
| Understand | L2 | Analyze | L4 | Create | L6 |

|  |  |  | BCLL | CO(s) | Marks |
| --- | --- | --- | --- | --- | --- |
| 1. | a) | What is the need for modulation? Classify the modulation schemes. | L2 | CO1 | [8M] |
|  | b) | Explain how AM signals can be demodulated using envelope detector. | L2 | CO1 | [7M] |
| 2. | a) | Explain the effect of frequency error in the local oscillator carrier in synchronous demodulation of SSB-SC signals. | L2 | CO2 | [8M] |
|  | b) | Mention the applications of different AM modulation methods shades. | L1 | CO2 | [7M] |
| 3. | a) | Explain Armstrong method for FM generation with an example. | L2 | CO3 | [8M] |
|  | b) | Explain FM demodulation using PLL. | L2 | CO3 | [7M] |
| 4. | a) | Derive the equation for figure of merit for DSB-SC modulation using coherent detector. | L3 | CO4 | [8M] |
|  | b) | Derive figure of merit equation for AM using envelope detector. | L3 | CO4 | [7M] |
| 5. | a) | What are the disadvantages of TRF receiver? Explain how these are addressed in super heterodyne receive. | L2 | CO5 | [8M] |
|  | b) | Write short notes on Automatic Gain Control. | L1 | CO5 | [7M] |
| 6. | a) | Compare different analog pulse modulation schemes. | L3 | CO6 | [8M] |
|  | b) | How a PPM signal can be generated from a PWM signal? Explain. | L2 | CO6 | [7M] |
| 7. | a) | Show that the maximum efficiency of AM is 66.66%. | L3 | CO1 | [5M] |
|  | b) | Explain, why SSB-SC is used for voice communication only? | L2 | CO2 | [5M] |
|  | c) | Compare FM and AM? | L3 | CO3 | [5M] |
| 8. | a) | Explain FM Receiver with a neat block diagram? | L2 | CO5 | [8M] |
|  | b) | Explain about Time Division Multiplexing with a neat block diagram? | L2 | CO6 | [7M] |

-- 00 -- 00 –