H.T No

Regulations:

A15

Sreenidhi Institute of Science and Technology

(An Autonomous Institution)

Code No: 5CC10    						                 	      Date: 09-July-2021 (AN)

B.Tech II-Year II- Semester Pending External Examination, July - 2021 (Supplementary)

Digital Signal Processing (ECE)

Time:	 3 Hours								                    Max.Marks:75

Note:    a)  No additional answer sheets will be provided.

b)  All sub-parts of a question must be answered at one place only, otherwise it will not be valued.

c)  Missing data can be assumed suitably.

ANSWER ANY 5 OUT OF 8 QUESTIONS. EACH QUESTION CARRIES 15 MARKS.

Bloom's Cognitive Levels of Learning (BCLL)

| Remember | L1 | Apply | L3 | Evaluate | L5 |
| --- | --- | --- | --- | --- | --- |
| Understand | L2 | Analyze | L4 | Create | L6 |

|  |  |  | BCLL | CO(s) | Marks |
| --- | --- | --- | --- | --- | --- |
| 1. | a) | A discrete time system is described by y[n]= y2[n-1] + x[n]. Determine whether the signal is stable or unstable for a bounded input x[n] = 2δ[n]. | L3 | CO1 | [8M] |
|  | b) | A discrete time system is given by y[n]=3y2[n-1]–5x[n]+4x[n-1]– 2x[n+1]. Is this system linear? Shift-invariant? Causal? In each case justify your answer. | L3 | CO1 | [7M] |
| 2. | a) | State and prove time shifting and frequency shifting properties of Discrete Fourier Series. | L1 | CO2 | [8M] |
|  | b) | Compute 4-point DFT of a sequence | L3 | CO2 | [7M] |
| 3. | a) | What is the need for FFT algorithm? State the computational requirements of Radix-2 FFT algorithm. | L2 | CO3 | [8M] |
|  | b) | Find the 8-point DFT of the sequence x[n] = {1, 2, 2, 1, 1, 2, 2, 1} using radix-2 Decimation In Frequency FFT algorithm. | L3 | CO3 | [7M] |
| 4. | a) | Obtain the Direct Form-I and Direct Form-II realizations of an IIR system described by y[n] – 1.6 y[n-1] + 0.25y[n-2] = x[n] + 0.5x[n-1] | L2 | CO4 | [8M] |
|  | b) | Draw the Cascade realizable structure for the FIR system defined by H(z) = (1 + 2z-1 - z-2) (1 + z-1 - z-2) | L1 | CO4 | [7M] |
| 5. | a) | Design a 2nd order Digital Butterworth filter with cut-off frequency of 1kHz and sampling frequency of 104 samples/sec by using Bilinear Transformation method. | L5 | CO5 | [8M] |
|  | b) | List the various window functions used for designing FIR Filter. | L1 | CO5 | [7M] |
| 6. | a) | What is the need for Multirate Digital signal Processing? List various applications of Multirate Digital Signal Processing systems. | L2 | CO6 | [8M] |
|  | b) | Explain interpolation process and draw the spectrum of interpolated signals and discuss the effect of imaging in the spectrum. | L2 | CO6 | [7M] |
| 7. | a) | Determine the impulse response for the cascade of two LTI systems having impulse responses, h1[n] = [1/2]n u[n] and h2[n] = [1/4]n u[n] | L4 | CO1 | [5M] |
|  | b) | Obtain the relation between DTFT and DFT. | L1 | CO2 | [5M] |
|  | c) | State and prove periodicity and symmetry properties of Twiddle Factor WN. | L1 | CO3 | [5M] |
| 8. | a) | Realize the following FIR system with minimum number of multipliers h[n] = { -0.5, 0.8, -0.5} | L3 | CO4 | [5M] |
|  | b) | Discuss the major differences between Butterworth and Chebyshev filters. | L2 | CO5 | [5M] |
|  | c) | How DSP processors are different from general purpose processors? | L2 | CO6 | [5M] |

-- 00 -- 00 –