H.T No

Regulations:

A15

Sreenidhi Institute of Science and Technology

(An Autonomous Institution)

Code No: 5AC43    						                 	      Date: 09-July-2021 (AN)

B.Tech II-Year II- Semester Pending External Examination, July - 2021 (Supplementary)

Electrical Technology (ECM)

Time:	 3 Hours								                    Max.Marks:75

Note:    a)  No additional answer sheets will be provided.

b)  All sub-parts of a question must be answered at one place only, otherwise it will not be valued.

c)  Missing data can be assumed suitably.

ANSWER ANY 5 OUT OF 8 QUESTIONS. EACH QUESTION CARRIES 15 MARKS.

Bloom's Cognitive Levels of Learning (BCLL)

| Remember | L1 | Apply | L3 | Evaluate | L5 |
| --- | --- | --- | --- | --- | --- |
| Understand | L2 | Analyze | L4 | Create | L6 |

|  |  |  | BCLL | CO(s) | Marks |
| --- | --- | --- | --- | --- | --- |
| 1. |  | Explain the construction features of a DC machine with the help of neat sketches. | L2 | CO1 | [15M] |
| 2. | a) | Which method preferred when speed of a DC shunt motor is to be controlled below rated speed and explain with help of diagram. | L2 | CO2 | [8M] |
|  | b) | A 220V DC shunt motor has an armature resistance of 0.5Ω and is excited the procedure constant flux at full load the motor runs at 1000rpm, taking an armature current of 40A. If a resistance of 0.8Ω is inserted in series with the armature determine speed at the full load. | L3 | CO2 | [7M] |
| 3. | a) | Explain the constructional details of a single phase transformer. | L2 | CO3 | [8M] |
|  | b) | A 4KVA, 200/400V, 50Hz single phase transformer is supplying full load current at 0.8 power factor. The following are the test results: O.C. test (lv side) : 200V, 0.8A, 70W (l.v. side) S.C. test (hv side): 20V, 10A, 60W (h.v. side) calculate efficiency, secondary voltage and current into primary at the above load. | L4 | CO3 | [7M] |
| 4. |  | Explain how the rotating magnetic field is developed in a 3-ø induction motor? | L2 | CO4 | [15M] |
| 5. | a) | Derive emf equation of synchronous generator | L3 | CO5 | [8M] |
|  | b) | Explain how voltage regulation of synchronous generator is determined using pessimistic method. | L2 | CO5 | [7M] |
| 6. | a) | Explain how the pulsating mmf of a 1-phase induction motor may be considered equivalent to two oppositely rotating fields. develop an expression for the torque of the motor. | L2 | CO6 | [8M] |
|  | b) | Explain how 1-phase motors are classified depending on construction & method of starting? | L2 | CO6 | [7M] |
| 7. | a) | Derive the expression for emf generated in a DC machine. | L3 | CO1 | [8M] |
|  | b) | Explain the operation principle of 3-point starter with neat diagram? | L2 | CO2 | [7M] |
| 8. | a) | Explain why a 3-phase induction motor cannot develop torque when running at synchronous speed. | L3 | CO4 | [8M] |
|  | b) | Calculate the synchronous speed, %slip and rotor frequency of three-phase, 50Hz,6-pole induction motor running at 970rpm. | L3 | CO5 | [7M] |

-- 00 -- 00 –