H.T No

Regulations:

A17

Sreenidhi Institute of Science and Technology

(An Autonomous Institution)

Code No: 6B515    						                 	      Date: 20-Mar-2021(FN)

B.Tech III-Year I- Semester External Examination, March/April - 2021 (Supplementary)

METAL CUTTING AND MACHINE TOOLS (ME)

Time:	 3 Hours								                    Max.Marks:75

Note:    a)  No additional answer sheets will be provided.

b)  All sub-parts of a question must be answered at one place only, otherwise it will not be valued.

c)  Missing data can be assumed suitably.

ANSWER ANY 5 OUT OF 8 QUESTIONS. EACH QUESTION CARRIES 15 MARKS.

Bloom's Cognitive Levels of Learning (BCLL)

| Remember | L1 | Apply | L3 | Evaluate | L5 |
| --- | --- | --- | --- | --- | --- |
| Understand | L2 | Analyze | L4 | Create | L6 |

|  |  |  | BCLL | CO(s) | Marks |
| --- | --- | --- | --- | --- | --- |
| 1. | a) | Draw a neat sketch of single point turning tool indicating its complete geometry on it. | L1 | CO1 | [8M] |
|  | b) | What are the various types of chips formed in metal cutting? Explain their conditions of formation with neat figures. | L1 | CO1 | [7M] |
| 2. | a) | What do you understand by the term “tool life” as applicable to metal cutting? Discuss the factors affecting the tool life. | L1 | CO2 | [8M] |
|  | b) | A tool life of 100 min is obtained from a cutting tool at a cutting speed of 25m/min and 10 min at a speed of 33.3m/min. what is the cutting speed for a 60min tool life? | L5 | CO2 | [7M] |
| 3. | a) | Draw the Lathe figure? Explain the lathe parts Briefly. | L1 | CO3 | [8M] |
|  | b) | Explain the method of turning screw threads on a lathe. | L2 | CO3 | [7M] |
| 4. | a) | Explain the parts of a Column and Knee type Milling machine with sketch. | L1 | CO4 | [8M] |
|  | b) | Distinguish between peripheral milling and face milling. | L3 | CO4 | [7M] |
| 5. | a) | How the Grinding Wheel is specified? Explain briefly. | L1 | CO5 | [8M] |
|  | b) | Explain the classification of grinding wheels. | L2 | CO5 | [7M] |
| 6. | a) | Write the applications of the following processes: (i) LBM (ii) ECM | L1 | CO6 | [9M] |
|  | b) | What are the main advantages of using jigs and fixtures in mass production? | L1 | CO6 | [6M] |
| 7. | a) | In an orthogonal turning operation the following data was obtained: cutting force = 120kg, feed force = 30kg, back rake angle = 150, feed rate = 0.2 mm/rev, chip thickness = 0.3 mm, cutting speed = 100 m/min, workpiece diameter = 120 mm, depth of cur = 0.4 mm. Calculate chip thickness ratio, shear angle, friction angle. | L5 | CO1 | [5M] |
|  | b) | What are the main characteristics a cutting tool material must possess? | L1 | CO2 | [5M] |
|  | c) | Explain any two drilling operations performed on drilling machine. | L1 | CO3 | [5M] |
| 8. | a) | What is principle of broaching operation? | L1 | CO4 | [5M] |
|  | b) | How does burnishing operation work? | L1 | CO5 | [5M] |
|  | c) | Write the working principle of Electron beam machining(EBM) . | L1 | CO6 | [5M] |

-- 00 -- 00 –