[image: image2.jpg]Sreenidhi Institute of Science and Tm:hnnlngy

AUTONOMOUS






Sreenidhi Institute of Science and Technology

(An Autonomous Institution)
Code No: 7B310  





                 
      Date: 30-Dec-2022 (FN)
B.Tech II-Year I- Semester (A18) Special Supplementary Examination - December-2022

MACHINE DRAWING AND COMPUTER AIDED DRAWING PRACTICE (ME)
Time:
 3 Hours







                    Max.Marks:70


 
Note:    a)  No additional answer sheets will be provided.

          

b)  All sub-parts of a question must be answered at one place only, otherwise it will not be valued.

          

c)  Missing data can be assumed suitably.

ANSWER ANY 5 OUT OF 8 QUESTIONS. EACH QUESTION CARRIES 14 MARKS.

Bloom's Cognitive Levels of Learning (BCLL)

	Remember
	L1
	Apply
	L3
	Evaluate
	L5

	Understand
	L2
	Analyze
	L4
	Create
	L6


ANSWER ALLTWO QUESTIONS. 

	1.
	
	ANSWER ANYTWO FROM QUESTION 1.
	BCLL
	CO(s)
	Marks

	
	a)
	Sketch the following  thread profiles for a nominal diameter of 25mm and pitch 3 mm show all the dimensions with suitable proportions
(i) Square Thread      (ii) ACME Thread    (iii) Buttress Thread
	L3
	CO2
	[15M]

	
	b)
	Draw the sectional front, top view of a cotter joint with socket and spigot ends two pipes diameter 30mm each. Show the proportions and suitable dimensions
	L3
	CO2
	[15M]

	
	c)
	Draw the full-sectional front view and top view of a double riveted chain lap joint for plates of 15mm thickness. Illustrate pitch, row pitch, diagonal pitch and margin.
	L4
	CO3
	[15M]


	2.
	Assemble all the parts shown in the figure

(i) Half sectional view from the front         (ii) Top view
	[40M]

	
	[image: image1.jpg]KNURLED

$14.5
®

8

.06

E |6 gl <1»24I
- ®

Parts list

06

®

$65

$65

HD,
8x7

SQT
DIA 3

Tommy bar

"SQTHD,

DIA 38x7





	


-- 00 -- 00 –
H.T No





Regulations:


A18











PAGE  
Page 1 of 2

