[image: image1.jpg]Sreenidhi Institute of Science and Tm:hnnlngy

AUTONOMOUS






Sreenidhi Institute of Science and Technology

(An Autonomous Institution)
Code No: 7B307   





                 
      Date: 04-Jan-2023 (FN)
B.Tech II-Year I- Semester (A18) Special Supplementary Examination - December-2022
MECHANICS OF SOLIDS (ME)

Time:
 3 Hours







                    Max.Marks:70


 
Note:    a)  No additional answer sheets will be provided.

          

b)  All sub-parts of a question must be answered at one place only, otherwise it will not be valued.

          

c)  Missing data can be assumed suitably.

ANSWER ANY 5 OUT OF 8 QUESTIONS. EACH QUESTION CARRIES 14 MARKS.

Bloom's Cognitive Levels of Learning (BCLL)

	Remember
	L1
	Apply
	L3
	Evaluate
	L5

	Understand
	L2
	Analyze
	L4
	Create
	L6


	
	 
	
	BCLL
	CO(s)
	Marks

	1.
	a)
	A steel rod 15m long is at a temperature of 150C. Find the free expansion of the length when the temperature is raised to 650C. Estimate the temperature stresses produced when the expansion of the rod is fully prevented . Take      E = 200GN/m2  and α = 12 X 10-6  per 0c.
	L3
	CO1
	[7M]

	
	b)
	A brass rod of 20mm diameter was subjected to a tensile force of 40kN. The extension of the rod was found to be 254 divisions in the 200mm extension meter. I each division is equal to 0.001mm , Calculate the elastic modulus of the brass.
	L3
	CO1
	[7M]

	
	
	
	
	
	

	2.
	a)
	Discuss the concept of Mohrs circle  .
	L1
	CO2
	[7M]

	
	b)
	The Principal stresses ata point across two perpendicular planes are 75MN/m2 and 35MN/m2 both tensile. Find the tangential and normal stresses.
	L3
	CO2
	[7M]

	
	
	
	
	
	

	3.
	
	A 6m long cantilever carries loads of 2kN and 3kN at 2m and 5m respectively from fixed end and a UDL of 10kN over its entire length .Calculate maximum SF and BM and Draw SF and BM diagrams for the same.
	L4
	CO3
	[14M]

	
	
	
	
	
	

	4.
	a)
	Explain the basic assumptions made in the theory of simple bending.
	L2
	CO4
	[7M]

	
	b)
	A beam consists of  a symmetrical rolled steel joist. The beam is simply supported at its ends and carries a point load at the centre of the span. If the maximum stress due to bending is 140MPa,Analyze the ratop pf the depth of the beam to span in order the central deflection may not exceed 1/480 of the span.
	L2
	CO4
	[7M]

	
	
	
	
	
	

	5.
	
	A cantilever beam 2 meters long is of rectangular section 100mm wide and 200mm deep. It carries  a uniformly distributed load of 2 kN per unit metre length for a length of 1.25metres from the fixed end and a point load of 0.8kN at the free end. Predict he deflection at the free end. Take E = 10GN/m2.
	L4
	CO5
	[14M]

	
	
	
	
	
	

	6.
	a)
	Explain the concept of the shrink fit in thick cylinders.
	L2
	CO6
	[7M]

	
	b)
	A cylindrical vessel whose ends are closed by means of rigid flange plates is made of steel plate 3mm thick. The internal length and diameter of vessel are 50cm and 25cm respectively. Determine the longitudinal and circumferential stresses in the cylindrical shell due to an internal pressure of 3MN/m2. Also calculate the increase in volume of the vessel.
	L2
	CO6
	[7M]

	
	
	
	
	
	

	7.
	a)
	Calculate the elongation of a bar due to its self weight.
	L2
	CO1
	[5M]

	
	b)
	Describe the assumptions in considering the Torsion of circular shafts.
	L2
	CO2
	[5M]

	
	c)
	Examine the boundary conditions of a SS beam.
	L2
	CO3
	[4M]

	
	
	
	
	
	

	8.
	a)
	Define the term Beams of uniform strength , mention its importance.
	L1
	CO4
	[5M]

	
	b)
	Explain the term flexural rigidity and discuss its importance .
	L1
	CO5
	[5M]

	
	c)
	Infer the assumptions made in the lames theorem for thick shells.
	L1
	CO6
	[4M]


-- 00 -- 00 –
H.T No





Regulations:


A18











PAGE  
Page 2 of 2

