[image: image2.jpg]Sreenidhi Institute of Science and Tm:hnnlngy

AUTONOMOUS






Sreenidhi Institute of Science and Technology

(An Autonomous Institution)
Code No: 5B101  





                 
      Date: 15-Sept-2023 (FN)
B.Tech I-Year I- Semester External Examination, September-2023 (Supplementary)
ENGINEERING DRAWING - I (Common to All)

Time:
 3 Hours







                    Max.Marks:75
 
Note:    a)  No additional answer sheets will be provided.

          

b)  All sub-parts of a question must be answered at one place only, otherwise it will not be valued.

          

c)  Missing data can be assumed suitably.

Bloom's Cognitive Levels of Learning (BCLL)
	Remember
	L1
	Apply
	L3
	Evaluate
	L5

	Understand
	L2
	Analyze
	L4
	Create
	L6









Part - A 



Max.Marks:25


[image: image1.wmf]Answer all QUESTIONS.
	
	
	BCLL
	CO(s)
	Marks

	1
	What is representative fraction? Illustrate with example.
	L1
	CO1
	[2M]

	2
	A line AB 90mm long is in the H.P. and makes an angle of 300 with the V.P. Its end A is 25mm in front of the V.P. Draw its projections.
	L3
	CO2
	[2M]

	3
	A 30mm side equilateral triangular plane is contained by VP and one of its base edge is parallel to HP and 30 mm above HP. Draw its projections.
	L3
	CO3
	[2M]

	4
	Draw the projections of a cone apex in the H.P. axis vertical and 40 mm in front of the V.P.
	L3
	CO4
	[2M]

	5
	What is the shape obtained, when the section plane is inclined to the axis and parallel to one of the generators of a cone?
	L3
	CO5
	[2M]

	6
	What are the engineering applications intersections of solids?
	L3
	CO6
	[3M]

	7
	Inscribe a Hexagon given the diameter of circle 40mm.
	L2
	CO1
	[3M]

	8
	Distinguish between horizontal and vertical trace. 
	L1
	CO3
	[3M]

	9
	What is true shape of a section?
	L1 
	CO5
	[3M]

	10
	Differentiate between a triangular pyramid and a tetrahedron

	L1
	CO4
	[3M]








           Part – B


   
 Max.Marks:50

ANSWER ANY FIVE QUESTIONS. EACH QUESTION CARRIES 10 MARKS.
	
	 
	
	BCLL
	CO(s)
	Marks

	11.
	
	A fixed point is 75mm from a fixed straight line. Draw the locus of a point P moving such a way that its distance from the fixed straight line is twice its distance from the fixed point. 
	L2
	CO1
	[10M]

	
	
	
	
	
	

	12.
	
	A line AB, 75 mm long, is inclined at 450 to the H.P. and 300 to the V.P. its end B is in the H.P. and 40 mm in front of the V.P. Draw its projections and determine its traces. 
	L2
	CO2
	[10M]

	
	
	
	
	
	

	13.
	
	Draw the projections of a regular hexagon of 25 mm side, having one of its sides in the H.P. and inclined at 600 to the V.P., and its surface making an angle of 450 with the H.P.
	L3
	CO3
	[10M]

	
	
	
	
	
	

	14.
	
	Draw the projections of a cone, base 60 mm diameter and axis 90mm long, lying on the H.P. on one of its generators with the axis parallel to the V.P.
	L3
	CO4
	[10M]

	
	
	
	
	
	

	15.
	
	A pentagonal pyramid, base 30 mm side and axis 65 mm long, has its base horizontal and an edge of the base parallel to the V.P. A horizontal section plane cuts it at a distance of 25 mm above the base. Draw its front view and sectional top view.
	L3
	CO5
	[10M]

	
	
	
	
	
	

	16.
	
	A vertical cone, diameter of base 75 mm and axis 100 mm long, is completely penetrated by a cylinder of 45 mm diameter. The axis of the cylinder is parallel to the H.P. and the V.P. and intersects the axis of the cone at a point 28 mm above the base. Draw the projections of the solids showing curves of intersection.
	L3
	CO6
	[10M]

	
	
	
	
	
	

	17.
	
	A line AB, 90 mm long, is inclined at 450 to the HP and its top view makes an angle of 600 with the VP. The end A is in the HP and 12 mm in front of the VP. Draw its front view and find its true inclination with the VP.
	L2
	CO1
	[10M]

	
	
	
	
	
	

	18.
	
	A cylinder of 40 mm diameter, 60 mm height and having its axis vertical, is cut by a sectional plane, perpendicular to the V.P., inclined at 450 to the H.P. and intersecting the axis 32 mm above the base. Draw its front view, sectional top view and true shape of the section.
	L3
	CO4
	[10M]


-- 00 -- 00 –
H.T No





Regulations:


A15











PAGE  
Page 1 of 2

_1604898669.unknown

