[image: image1.jpg]Sreenidhi Institute of Science and Tm:hnnlngy

AUTONOMOUS






Sreenidhi Institute of Science and Technology

(An Autonomous Institution)
Code No: 8CC55 





                 
      Date: 17-Aug-2023 (FN)
B.Tech II-Year II- Semester External Examination, Aug - 2023 (Regular & Supplementary)
DIGITAL ELECTRONICS (CSE, IT, CS, AIML, DS and IOT)
Time:
 3 Hours







                    Max.Marks:70


 
Note:    a)  No additional answer sheets will be provided.

          

b)  All sub-parts of a question must be answered at one place only, otherwise it will not be valued.

          

c)  Missing data can be assumed suitably.

Bloom's Cognitive Levels of Learning (BCLL)

	Remember
	L1
	Apply
	L3
	Evaluate
	L5

	Understand
	L2
	Analyze
	L4
	Create
	L6








Part - A 




Max.Marks:20
Answer all QUESTIONS

	
	
	BCLL
	CO(s)
	Marks

	1
	Convert the following to the decimal form.

 i) (01101001.101)2   

ii) (A23B)16
	L3
	CO1
	[2M]

	2
	Obtain dual of the expression AB+A(C+AC+BC.
	L2
	CO2
	[2M]

	3
	Draw a full subtractor logic diagram with truth table.
	L4
	CO3
	[2M]

	4
	Write the properties of Bistable latch.
	L2
	CO4
	[2M]

	5
	Write the applications of flip flops.
	L2
	CO5
	[2M]

	6
	Differentiate between ROM, PROM.
	L4
	CO6
	[2M]

	7
	Explain the Properties of XOR Gate.
	L2
	CO1
	[2M]

	8
	What are the differences between flip flop and latch
	L1
	CO4
	[2M]

	9
	Differentiate between Synchronous and Asynchronous counter.
	L4
	CO5
	[2M]

	10
	Write the characteristics of PAL and PLA.

	L2
	CO6
	[2M]








Part – B 




Max.Marks:50
ANSWER ANY FIVE QUESTIONS. EACH QUESTION CARRIES 10 MARKS.
	
	 
	
	BCLL
	CO(s)
	Marks

	11.
	a)
	Convert (A1F9)16 to decimal, binary, octal.
	L3
	CO1
	[5M]

	
	b)
	Realize AND,OR gates from universal gates.
	L4
	CO1
	[5M]

	
	
	
	
	
	

	12.
	a)
	Minimize the function f(A,B,C,D)=((1,4,6,10,14)+d(0,8,11,15) 
using K-map.
	L4
	CO2
	[5M]

	
	b)
	Obtain the compliment of the following Boolean expressions

 i) A(B+A(BC(+A(BCD+A(BC(D(E        ii) ABEF+ABE(F(+A(B(EF.
	L4
	CO2
	[5M]

	
	
	
	
	
	

	13.
	a)
	Explain the differences between a MUX and a DEMUX.
	L4
	CO3
	[5M]

	
	b)
	Realize a full adder using half adders and explain the truth table.
	L4
	CO3
	[5M]

	
	
	
	
	
	

	14.
	a)
	Design Master -slave JK flipflop with necessary diagrams.
	L4
	CO4
	[5M]

	
	b)
	Convert JK flip flop to T flip flop.
	L2
	CO4
	[5M]

	
	
	
	
	
	

	15.
	a)
	Design a 4 bit ring counter.
	L4
	CO5
	[5M]

	
	b)
	Design a 4 bit up ripple counter and explain its timing diagram.
	L4
	CO5
	[5M]

	
	
	
	
	
	

	16.
	a)
	Implement f(A,B,C,D) =Σm(0,1,4,5,6,7,9,10,12,13,15) using PLA and explain its procedure. 
	L4
	CO6
	[5M]

	
	b)
	Explain the memory organization with a neat diagram.
	L4
	CO6
	[5M]

	
	
	
	
	
	

	17.
	a)
	Given two binary numbers X=1010100 and Y=1000011.

Perform    i)X-Y      ii)Y-X using 2’s complement method.
	L3
	CO1
	[4M]

	
	b)
	Obtain dual of the following Boolean expressions      
i) AB+A(B+C) +B((B+D)        ii) A+B+A(B(C.
	L3
	CO2
	[3M]

	
	c)
	Realize3-bit digital comparator and explain the truth table.
	L4
	CO3
	[3M]

	
	
	
	
	
	

	18.
	a)
	Differentiate SR-FF and JK-FF with their functional operation and excitation tables.
	L4
	CO4
	[4M]

	
	b)
	What do you mean by universal shift register? Draw and explain its circuit diagram and operation.
	L1
	CO5
	[3M]

	
	c)
	Differentiate between ROM and RAM.

	L4
	CO6
	[3M]


-- 00 -- 00 –
H.T No





Regulations:


A20











PAGE  
Page 2 of 2

