H.T No

Regulations:

A20

Sreenidhi Institute of Science and Technology

(An Autonomous Institution)

Code No: 8K394   					                 	  Date: 11-Sept-2024  (FN)

B.Tech II-Year I- Semester External Examination, Sept-2024 (Supplementary)

COMPREHENSIVE TEST AND VIVA VOCE-III  (CIVIL)

Time:	3:00 Hours								                    Max.Marks:70

Note: Answer Any 70 Questions.

Student can solve more than 70 questions, but maximum 70 marks will be awarded.

Calculator's are not allowed.

| S.No | Question |
| --- | --- |
| 1. | The moment diagram for a cantilever whose free end is subjected load to a bending moment, will be (A) Triangle (B) Rectangle (C) Parabola (D) Cubic parabola |
| 2. | During a tensile test on a specimen of 1 cm cross-section, maximum load observed was 8 tonnes and area of cross-section at neck was 0.5. cm2. Ultimate tensile strength of specimen is (a) 4 tonnes/cm2 (b) 8 tonnes/cm2 (c) 16 tonnes/cm2 (d) 22 tonnes/cm2 |
| 3. | For steel, the ultimate strength in shear as compared to in tension is nearly (a) same (b) half (c) one-third (d) two-third |
| 4. | Which of the following has no unit (a) kinematic viscosity (b) surface tension (c) bulk modulus (d) strain |
| 5. | The percentage reduction in area of a cast iron specimen during tensile test would be of the order of (a) more than 50 % (b) 25-50% (c) 10-25% (d) negligible. |
| 6. | If a material expands freely due to heating it will develop (a) thermal stresses (b) tensile stress (c) bending (d) no stress |
| 7. | In a tensile test, near the elastic limit zone, the (a) tensile strain increases more quickly (b) tensile strain decreases more quickly (c) tensile strain increases in proportion to the stress (d) tensile strain decreases in proportion to the stress |
| 8. | The stress necessary to initiate yielding is (a) considerably greater than that necessary to continue it (b) considerably lesser than that necessary to continue it (c) greater than that necessary to stop it (d) lesser than that necessary to stop it |
| 9. | The strain energy stored in a body due to suddenly applied load compared to when it is applied gradually is (a) same (b) twice (c) four times (d) eight times |
| 10. | A material capable of absorbing large amount of energy - before fracture is known as (a) ductility (b) toughness (c) resilience (d) shock proof (e) plasticity. |
| 11. | In a beam, the neutral plane (A) May be its centre (B) Passes through the C.G. of the area of cross-section (C) Does not change during deformation (D) None of these |
| 12. | A beam is loaded as cantilever. If the load at the end is increased, the failure will occur (a) in the middle (b) at the tip below the load (c) at the support (d) anywhere |
| 13. | A beam is said to be of uniform strength, if (A) B.M. is same throughout the beam (B) Deflection is same throughout the beam (C) Bending stress is same throughout the beam (D) Shear stress is same throughout the beam |
| 14. | The ratio of limiting friction and normal reaction is known as (A) Coefficient of friction (B) Angle of friction (C) Angle of repose (D) Sliding friction |
| 15. | The under mentioned type is simple strain (A) Tensile strain (B) Compressive strain (C) Shear strain (D) All the above |
| 16. | The maximum frictional force which comes into play when a body just begins to slide over another surface is called (A) Limiting friction (B) Sliding friction (C) Rolling friction (D) Kinematic friction |
| 17. | The stress necessary to initiate yielding, is considerably (A) More than that necessary to continue it (B) Less than that necessary to continue it (C) More than that necessary to stop it (D) Less than that necessary to stop it |
| 18. | The units of moment of inertia of an area are (A) kg m2 (B) m4 (C) kg/m2 (D) m3 |
| 19. | A force acting on a body may (A) Introduce internal stresses (B) Balance the other forces acting on it (C) Retard its motion (D) All of the above |
| 20. | Effect of a force on a body depends upon (A) Magnitude (B) Direction (C) Position or line of action (D) All of the above |
| 21. | Express the quadrant bearing S 31° 36’E of line in WCB - 121°36’ b) 148°24’ c) 238°24’ d) 221°36’ |
| 22. | The horizontal angle between the true meridian and magnetic at a place is called a)The magnetic bearing b) Magnetic declination c)Local attraction d) azimuth |
| 23. | local attraction is any influence which prevents the needle from pointing to _____ in a given locality is called a)the magnetic north b)the magnetic south c)true north d)true south |
| 24. | The smaller horizontal angle between the true meridian and a survey line, is known a)Declination b) Bearing c) Azimuth d) Dip |
| 25. | Survey starts at one point and ends at some other point a) open traverse b) Closed traverse c) Line of collimation d) Swinging |
| 26. | The method used to measure horizontal angle to a finer degree of accuracy than that obtainable with least count the Vernier a) Repetition method b) Closed traverse c) Line of collimation d) Swinging |
| 27. | The method used to measure horizontal angle is suitable for a group of angle having a common vertex point a) Repetition method b) Reiteration method c) Line of collimation d) Open traverse |
| 28. | Straight line tangential to the longitudinal curve of the level tube at the centre of the tube and it is horizontal when the bubble is in the centre a) Axis of Level tube b) Axis of telescope c) zenith axis d) none of above |
| 29. | The EDM instrument is based on generation , propagation, reflection and reception of a)Light waves b) electromagnetic waves c)sound waves d) none of the above |
| 30. | Which type of sources of EM energy used by Positive remote sensing a)Sun b) own sources c) moon d) none |
| 31. | Which type of sources of EM energy used by active remote sensing a)Sun b) own sources c) moon d) none |
| 32. | GIS stands for a) Geographic Information System b) Generic Information System c) Geological Information System d)Geographic Information Sharing |
| 33. | The interaction of the electromagnetic radiation produced with a specific wave length to illuminate a target on the terrain for studying its scattered radiance, is called: a)passive remote sensing b) active remote sensing c)neutral remote sensing d) None of these |
| 34. | Repetitive observations of the same area at equal interval of time, are useful to monitor the dynamic phenomena: a)Lithology b) Vegetative cover c) Buildings d) Groundwater |
| 35. | Consider the following statements regarding the satellite imaging: - The satellite orbit is fixed in the inertial space - During successive across-track imaging, the earth rotates beneath the sensor - The satellite images a skewed area Which one of the following is correct regarding the above statements? # a)1, 2, 3 b) 1, 3 c) 2, 3 d) 1, 2 |
| 36. | The altitudinal distance of a geostationary satellite from the earth is about: a)26, 000 km b) 30, 000 km c) 36, 000 km d) 44, 000 km |
| 37. | How much inclination must be provided in a tilted photograph? a) 13˚ b) 20˚ c) 3˚ d) 34˚ |
| 38. | If the apparent horizon is shown in a photograph, it is low oblique. a) True b) False |
| 39. | Perspective projection is produced from__________ a) Straight lines radiating a common point b) Straight lines radiating different points c) Parallel lines radiating a common point d) Perpendicular lines radiating a common point |
| 40. | Flying height refers to_________ a) Upper portion of the exposure station b) Bottom of the exposure station c) Depression of the exposure station d) Elevation of the exposure station |
| 41. | Self centered, self defense, self discipline, self esteem, self pity are examples of--- a) adverbs b) verbs c) nouns d) adjectives |
| 42. | In SWOT analysis Grid , what stands for S a) Strength b) Specific c)Smart d) Sensible |
| 43. | Time Management is Important to a) Every one b) Employees only c) Students only d) Nom one |
| 44. | Failing to manage your time can lead to some consequences like a) Less stress b) Missed deadlines c) Greater productivity d) A better Professional reputation |
| 45. | Sentence correction: He suggested that, we should be bound by a code of conduct, isn't it ? a) shouldn't he ? b) wasn't he? c) shouldn't we ? d) is he ? |
| 46. | Sentence correction: The furniture in this room are made of teak. a) was b) is c) has d) am |
| 47. | Select the element that is not part of the critical thinking standards. a) clarity b) relevance c) Relativistic thinking d) completeness |
| 48. | What is not the characteristic of a critical thinker? a) He thinks independently and does not always succumb to peer pressure. b) He upholds the standards of critical thinking. c) He refuses to recognize the limitations of his mind and consistently pursues excellence. d) He uses logical skills in reasoning |
| 49. | ## Ravi works hard to get the best student award at the end of year. What type of motivation is this? a) Internal b) External c) immediate d) None of the above |
| 50. | Which of these is not a step in the preparation of an interview? a) Analyzing yourself b) identifying your skills c) Being negative d) Revising your subject |
| 51. | Physical facilities are -------- & --------- for animals a) non essential & complete b) necessary & incomplete c) necessary & complete d) none of the above |
| 52. | The state in which a human being possesses more facilities than what are required is called a) happiness b) prosperity c) satisfaction d) all of these |
| 53. | Which is not the models of professional role - saviour b) selfish c) bureaucratic servant d) game player |
| 54. | The needs of the self are ensured by ______ a) right feeling b) right understanding c) both d) none |
| 55. | We are all similar at the level of our ___ but differ in our___ a) Intension, Competence b) guidance, respect c) affection, glory d) Competence, Intension |
| 56. | Society is an extension of a) Family b) Nature c) self d) work |
| 57. | Our natural acceptance is to live in ------- with nature a) harmony b) opposition c) struggle for survival d) none of these |
| 58. | Conformance of human order is named as a) right value/sanskar b) constitution conformance c)breed conformance d) seed conformance |
| 59. | Values important for relationship are many they may include: a)Aggression b) Competition c)Integrity and character d) Arrogance |
| 60. | Which of the following are true with regard to safety and treatment of employees a) It is a means of promoting worker efficiency and social order by protecting lives and promoting the well-being of workers b) The idea of safety and treatment of employees was conceived during the Industrial Revolution in Europe c) All workers have a right to expect fairness from their employer and to be treated with respect and dignity d) ALL |
| 61. | The value of where c: is will be a) 0 b) c) d) |
| 62. | If then z=0 is its…. (a) Removable singularity ( b) Isolated singularity (c) Essential singularity (d) None of these |
| 63. | Expand the function in Taylors series about z=1 a) b) c) d) None |
| 64. | The simple poles of are a) 1,4 b) -1,4 c) d)None |
| 65. | The sum of residues at its poles of is a) 1 b) 0 c) d) None |
| 66. | For large samples population mean = ______________ (a) E(x) (b) E() (c) Var (1) (d) Var () |
| 67. | A statistic is said to be __________________ estimation, if E (statistic) = Parameter. (a) Biased (b) Un-biased (c) Consistent (d) Inconsistent |
| 68. | The Confidence interval for Population mean µ at 95% are ________________ (a) 2.58 (b) 1.96 (c) 2.33 (d) 1.65 |
| 69. | The number of control charts for attributes. a) 1 b) 2 c) 3 d) 4 |
| 70. | What is the central limit for C chart. a) C b) Mean of C c) X d) Mean of X |
| 71. | What is the central limit for Range chart. a) C b) Mean of C c) R d) Mean of R |
| 72. | In φ² - test the df without estimating population parameters from sample statistics is r is - n (b) k-1 (c) K (d) k + 1 |
| 73. | If there are M number of Population Parameters from sample statistics then r is K-1 (b) K (c) K+1 (d) K-1-M |
| 74. | To test the independence of attributes we use ____ test (a) φ² – test (b) test for specified Means (c) test diff. of 2 Means (d) Paired t – test |
| 75. | What will be the value of the following Python expression? 3*(6-2)/2+7 a)13 b)11 c)1.3 d)12 |
| 76. | Which of the following character is used to give single-line comments in Python? a) // b) # c) ! d) /* |
| 77. | Which of the following functions is a built-in function in python? a) factorial() b) print() c) seed() d) sqrt() |
| 78. | ____________ method is used to display widgets like button, label, Entry on the main window. (A) Button (B) pack (C) Label (D) grids |
| 79. | ____________ is the time complexity of Quick sort in worst case where N is the size of the list. (A)O(logN) (B)O(2N) (C) O(N2) (D) O(N logN) |
| 80. | ___________ problem is solved by dynamic programming. A) Binary search B) Merge Sort C) Minimum cost spanning tree D) Optimal Binary search tree |
| 81. | A badly mixed cement concrete results in a) segregation b) bleeding c) honey combing d) none of these |
| 82. | Pick up the correct statement regarding low heat cement from the following: a) It possesses less compressive strength b) Its initial setting time is about one hour c) Its final setting time is about 10 hours d) Its mainly used for mass concrete work e) All of the above |
| 83. | Chemically, marble is known as a) metamorphic rock b) argillaceous rock c) calcareous rock d) silicious rock. |
| 84. | Inner part of a timber log surrounding the pith, is called a) sapwood b) cambium layer c) heart wood d) none of these |
| 85. | Shingle is a) decomposed laterite b) crushed granite c) water bound pebbles d) air weathered rock. |
| 86. | Pick up the plutonic rock from the following: a) Granite b) Dolerite c) Basalt d) All the above. |
| 87. | The lime which contains mainly calcium oxide and slacks with water, is a) fat lime b) quick lime c) hydraulic lime d) poor lime |
| 88. | Seasoning is a) a process of removing sap b) creosoting c) painting with sodium silicate d) coating with tar. |
| 89. | If the iron ore contains clay as an impurity, the flux added during calcination, is a) clay b) lime stone c) argillaceous iron ore d) all the above |
| 90. | Cement is said to be of good quality if a) its colour is not greenish grey b) one feels cool by thrusting one's hand in the cement bag c) it is not smooth when rubbed in between fingers d) a handful of cement thrown into a bucket of water does not float e) none of these |
| 91. | Cast steel is manufactured by a) Cementation process b) Crucible process c) Bessemer process d) Open hearth process. |
| 92. | For the manufacture of plywood, veneers are placed so that grains of adjacent veneers a) run at right angles b) parallel c) inclined at 45° d) inclined at 60°. |
| 93. | The timber having maximum resistance against white ants, is obtained from a) chir b) shisham c) sal d) teak |
| 94. | Knots in timber are a) defects caused by crushing fibres b) splits radiating from the centre c) speckled strains d) signs of branches cut off. |
| 95. | Pick up the correct statement from the following: a) Solder material is an alloy which melts at a temperature above 400°C b) Brazing is done at temperature above 600°C to 1100°C c) Brazing joint is stronger than the solder joint d) all the above |
| 96. | Lime concrete is generally used for a) wall foundations b) flooring at ground level c) both (a) and (b) d) neither (a) nor (b). |
| 97. | Smith's test of stones is performed to find out a) the presence of soluble matter of stone b) the compressive strength of the stone c) the hardness of the stone d) the toughness of the stone. |
| 98. | The cracks caused by shrinkage of the exterior surface of the wood exposed to atmosphere, are called: a) radial shakes b) heart shakes c) wind cracks d) twisted fibres. |
| 99. | The process of decarbonising the pig iron completely and then adding proper percentage of carbon for manufacturing steel, is called a) Cementation process b) crucible process c) Bessemer process d) Open hearth process. |
| 100. | In paints, the pigment is responsible for a) durability b) colour c) smoothness d) glassy face e) none of these |

-- 00 -- 00 –