H.T No

Regulations:

A20

Sreenidhi Institute of Science and Technology

(An Autonomous Institution)

Code No: 8C466   					                 	      Date:28-August-2024(FN)

B.Tech II-Year II- Semester External Examination, August - 2024 (Supplementary)

COMPREHENSIVE TEST AND VIVA VOCE-IV (ECE)

Time:	3:00 Hours								                    Max.Marks:70

Note: Answer Any 70 Questions.

Student can solve more than 70 questions, but maximum 70 marks will be awarded.

Calculator's are not allowed.

| S.No | Question |
| --- | --- |
| 1. | A simple parity check code can detect a)odd number of errors b)even number of errors c)positive number of errors d) negative number of errors |
| 2. | What is the minimum Hamming distance between the code words 1010 and 0111 a)1 (b) 2 (c) 3 (d) 4 |
| 3. | The cyclic codes are designed using a)Shift registers with feedback b)Shift registers without feedback c)Flip Flops d)Gates |
| 4. | In the dividing circuit, the parity polynomial is obtained by the a) Quotient (b) Remainder (c) Dividend (d) Divisor |
| 5. | The Gaussian surface for a point charge will be a) Cube b) Cylinder c) Sphere d) Cuboid |
| 6. | Find the work done moving a charge 2C having potential V = 24volts a) 96 b) 24 c)36 d) 48 |
| 7. | Dipole moments are used to calculate the a) Electric field intensity b)Polarization patterns c) Strength of the dipole in the field d) Susceptibility |
| 8. | The electrostatic energy in an electric field does not depend on which of the following? a) Magnitude of charges b) Permittivity c) Applied electric field d) Flux lines |
| 9. | Ampere law states that a) Divergence of H is same as the flux b) Curl of D is same as the current c) Divergence of E is zero d) Curl of H is same as the current density |
| 10. | Identify which of the following is the unit of magnetic flux density? a) Weber b) Weber/m c) Tesla d) Weber-1 |
| 11. | The divergence of H will be a) 1 b) -1 c) ∞ d) 0 |
| 12. | If ∫ H.dL = 0, then which statement will be true? a) E = -Grad(V) b) B = -Grad(D) c) H = -Grad(Vm) d) D = -Grad(A) |
| 13. | In the conversion of line integral of H into surface integral, which theorem is used? a) Green theorem b) Gauss theorem c) Stokes theorem d) It cannot be converted |
| 14. | Find the equation of displacement current density in frequency domain a)Jd = jωεE b)Jd = jωεH c) Jd = ωεE/j d) Jd = jεE/ω |
| 15. | Both the conduction and displacement current densities coexist in which medium? a)Only conductors in air b) Only dielectrics in air c) Conductors placed in any dielectric medium d) Both the densities can never coexist |
| 16. | Which quantity is solenoidal in the electromagnetic theory? a) Electric field intensity b) Electric flux density c) Magnetic field intensity d)Magnetic flux density |
| 17. | In conductors, which condition will be true? a) σ/ωε> 1 b) σωε> 1 c) σ/ωε< 1 d) σωε< 1 |
| 18. | The average power in an electromagnetic wave is given by a) propagation constant b) pointing vector c) phase constant d) attenuation constant |
| 19. | At high frequencies, which parameter is significant? a) Conduction current b) Displacement current c) Attenuation constant d) Phase constant |
| 20. | The expression for the loss tangent is given by a) σ/ωε b) ωε/σ c) σ/ω d) ω/ε |
| 21. | The standing wave ratio of short circuited and open circuited lines will be a) 0 b) 1 c) -1 d) ∞ |
| 22. | For matched line, the standing wave ratio will be a) 0 b) ∞ c) -1 d) 1 |
| 23. | The condition that holds good in a distortion less transmission line is a) R/L = G/C b) RL = GC c) L/R = G/C d) RG/LC |
| 24. | Which two parameters given below are zero in the lossless line? a) L, C b) C, G c) G, L d) R, G |
| 25. | The normalized load impedance of the transmission line 50 ohm with a load of 30 ohm is a) 30 b) 150 c) 5/3 d) 3/5 |
| 26. | The characteristic impedance of a line having open and short impedances of 20 and 5 respectively is a) 20 b) 100 c) 25 d) 10 |
| 27. | The maxima and minima voltage of the standing wave are 6 and 2 respectively. The standing wave ratio is a) 2 b) 3 c) ½ d) 4 |
| 28. | The range of the standing wave ratio is a) 0 < S < 1 b) -1 < S < 1 c) 1 < S < ∞ d) 0 < S < ∞ |
| 29. | The CMRR of op-amp is given by a) Ac/Ad b) Ad/Ac c) Ac+Ad d) Ac-Ad |
| 30. | The current drawn by the two input terminals of op-amp a) 0 b) 1 c)-1 d) ∞ |
| 31. | The gain of Op-amp voltage follower is - 0 b) 1 c)-1 d) ∞ |
| 32. | The open loop voltage gain of an Op-Amp is - 0 b) 1 c)-1 d) ∞ |
| 33. | One of the application of op-amp comparator is - Zero crossing detector (b) Peak detector c)clipper (d) clamper |
| 34. | Name the non-linear circuit which is also called as DC Restorer a) Integrator b) Clamper c) Clipper d) Differentiat |
| 35. | V-I Converters are also called as (a) Transresistance amplifier (b) Voltage amplifier c)Current amplifier (d) Transconductance amplifier |
| 36. | What will happened in ideal integrator for DC input - Behaves as a buffer (b) Behaves as a comparator c) Behaves an a Low pass circuit (d) Behaves as a High pass circuit |
| 37. | IC566 is a - Phase locked loop b) Vollage controlled oscillator c) Schmitt trigger d)Astable multivibrator |
| 38. | Duty cycle of astable multivibrator is given by a)D=Ton/Ttotal time b) D=Toff/Ttotal time c)D=Ttotal time/Ton d) D=Ttotal time/Toff |
| 39. | The pass band gain of second order filters is given by a)Ao=1.586 b) Ao=0.586 c) Ao=1.568 d) Ao=0.568 |
| 40. | The Quality factor of Wide band pass filter is A)Q=10 b) Q<10 c)Q>10 d)None of the above |
| 41. | The primary disadvantage of the flash analog-to digital converter (ADC) is that a) more comparators b) long conversion time c) more voltage d) none |
| 42. | The difference between analog voltage represented by two adjacent digital codes, or the analog step size, is the a) quantization b) accuracy c) resolution d) monotonicity |
| 43. | What is IC 723? a) A voltage regulator b) op-amp c) timer d) none |
| 44. | The 7812 regulator IC provides ________. a) 5 V b) –5 V c) –12 V d) 12 V |
| 45. | ECL strands for -------logic a) emitter capacitor b) emitter coupled c) electronics couple d) none |
| 46. | The time needed for an output to change from the result of an input change is known as: a) propagation delay b) noise immunity c) fan-out d) rise time |
| 47. | The problem of interfacing IC logic families that have different supply voltages (VCC's) can be solved by using a) level-shifter b) tristate shifter c) decoupling capacitor d) pull-down resistor |
| 48. | What is the advantage of using low-power Schottky (LS) over standard TTL logic? - more power dissipation b) less power dissipation c) cost is less d) cost is less |
| 49. | 74HC00 IC HC stands for A) high speed cmos b) highpower cmos c)hightenson compatible d) none |
| 50. | CMOS ICS are availabe with following series a) 2000 b) 1000 c) 4000 d) 8000 |
| 51. | The time needed for an output to change from the result of an input change is known as: a)propagation delay b)rise time c)fall time d)peak change |
| 52. | A level-shifter circuit needed in interfacing logic when a) always b) never c) supply voltages are the same d) supply voltages are the same |
| 53. | Class ________ amplifiers are normally operated in a push-pull configuration in order to produce an output that is a replica of the input. a)A b) B c)AB d) C |
| 54. | You find that there is an input signal on the base of Q1 and Q2. However, there is no output signal. You then measure the dc voltages on Q2 and find them to be all 0V. The possible trouble might be a) C3 is shorted. b) C1 is open. c) RL is shorted. d) VCC is 0V. |
| 55. | Which type of power amplifier is biased for operation at less than 180º of the cycle? a) A b) B c)C d) AB |
| 56. | Class AB operation is ________ operation. a)s imilar to class A b) similar to class B c) similar to class C d) None of the above |
| 57. | At parallel resonance, the net reactive component circuit current is __________ a) Capacitive b) Zero c) Inductive d) None of the above |
| 58. | At parallel resonance, the phase angle between the applied voltage and circuit current is ____ a) 90o b) 180o c) 0o d) None of the above |
| 59. | At parallel resonance, the ratio L/C is __________ a) Very large b) Zero c) Small d) None of the above |
| 60. | At series or parallel resonance, the circuit behaves as a __________ load a) Capacitive b) Resistive c) Inductive d) None of the above |
| 61. | A limiter circuit is a __________ circuit. a) clipper b) wave shaping c) clamper d) a & b are correct |
| 62. | Slicer, Amplitude Selector and Limiter are the other names of clipping circuit. a) True b) False c) both d) none |
| 63. | A biased clipper is used to limit the ________ of a waveform at a level other than 0V. a) frequency b) amplitude c) time d) intensity |
| 64. | The circuit which removes positive level of input signal is called positive clipper. a) True b) False c) both d) none |
| 65. | The pulse width of a monostablemultivibrator is given by a) 0.693 RC b) 1.386 RC c) 2 RC d) 5 RC |
| 66. | Which multivibrator is called a free- running circuit? a)Bistable b)monostable c) astable d) Schmitt trigger |
| 67. | Which multivibrator is not require external triggering signal? a)Bistable b)monostable c) astable d) none of these |
| 68. | The time period of an astablemultivibrator is a) 0.693R1C1 b) 0.693R2C2 c) 0.693(R1C1+R2C2) d) 1.386RC |
| 69. | The application of a voltage time base generator is in a measuring instrument like: a) CRO b) Frequency counters c)Digital voltmeter d) Multimeter |
| 70. | Errors es, ed and et define a) Current gain b) Voltage gain c) Deviation from linearity d) Bandwidth |
| 71. | If in a simple exponential sweep generator, the supply voltage is 200V and the required sweep amplitude is 10V, and then the slope a) 5% b) 20% c) 200% d) 0.5% |
| 72. | The sweep duration of a UJT sweep generator is given as: a) 0.69 τ b) τ ln{(VBB − VP)/(VBB − VV )} c)τ ln{(VBB − VV )/(VBB − VP)} d) 1.4 τ |
| 73. | The maximum number of inputs a logic gate has is called a) Fan-out b) Fan-in c) Figure of merit d) Totem-pole output |
| 74. | The time required for a logic gate to switch from a low output state to a high output state and vice-versa is called as: a) Propagation delay b)Rise time c) Transition time d) Turn-on time |
| 75. | TTL gates are preferred because of their a) Lesser power dissipation b)Faster switching speed c) Manufacturing process is simple d) Weight is less |
| 76. | The NOR gate is OR gate followed by ………… a) AND gate b) NAND gate c) NOT gate d) none of these |
| 77. | Any activity aimed at earning or spending money is called? a) Service b) Accounts c)Economic d) Physics |
| 78. | Which costs remains constant in the short & Long run? (a) Historical (b) Fixed cost (c) Future cost (d) Variable cost |
| 79. | Which is one of the conventions of accounting? a)Business entity b) Money Measurement c) Accounting period d) Conservatism |
| 80. | Trading Account is prepared to find out? a) Gross Profit b) Gross Loss c) Both A& B d) Net profit |
| 81. | Which of the following Amplitude modulation schemes has least bandwidth a) AM (b) DSB-SC (c) SSB-SC (d) VSB-SC |
| 82. | Which of the following modulation schemes use Coherent detector as the demodulator (a) DSB-SC (b) SSB-SC (c) VSB-SC (d) All the above |
| 83. | The feedback path in the Coastas receiver is used to reduce the A)Amplitude drift in the carrier B) Frequency drift in the carrier C)Both (a) & (b) D)None |
| 84. | An envelope detector is used to demodulate a) AM (b) DSB-SC (c) SSB-SC (d) FM |
| 85. | Bandwidth of Narrow Band FM is a) fm (b) 2fm (c) 2fm(β+1) (d) fm(β+1) |
| 86. | Bandwidth of Wide Band FM is a) fm (b) 2fm (c) 2fm(β+1) (d) fm(β+1) |
| 87. | In Narrow Band FM, the modulation index is a) Less than 1 (b) Greater than 1 (c) Zero (d) Infinite |
| 88. | In Frequency Modulation, the amount of frequency deviation of the carrier frequency is - dependent onModulating signal amplitude b)Modulating signal frequency c)Both (a) and (b) d)None |
| 89. | The dynamic range of a super heterodyne receiver is improved with the help of a)RF stage (b) Mixer (c) IF stage (d) AGC |
| 90. | For an AM receiver tuned to 1000KHz, the image frequency is obtained at (a) 455KHz (b) 1455KHz (c) 545KHz (d) 1910KHz |
| 91. | Pre-emphasis circuit is used in which of the following transmitters a) AM (b) FM (c) PAM (d) PCM |
| 92. | The fidelity of the receiver is the ability to recover (a) message signal (b) carrier signal (c) Image signal (d) None |
| 93. | The bandwidth required in DPCM is less than that of PCM because (a) the number of bits per code is reduced resulting in a reduced bit rate (b) more quantization levels are needed (c) the difference signal is larger in amplitude than actual signal (d) the successive samples of signal often differ in amplitude |
| 94. | The bandwidth requirement of a telephone channel is (a) 5KHz (b) 15KHz (c) 3KHz (d) 25KHz |
| 95. | A 5-bit PCM encoder is used. On account of modulation, voltage involved is 27.39V. The quantization error is (a) 0.39V (b) 0.6V (c) 0.09V (d) 0.428V |
| 96. | The problems due to uniform step-size are overcome by (a) PDM (b) PCM (c) ADM (d) DPCM |
| 97. | Modems are used for (a) modulating digital signals (b) converting analog to digital and vice-versa (c) either of (a) or (b) (d) none of these |
| 98. | The full form of ISI is (a) Inter Symbol Interference (b) Inter System Interference (c) Intra Symbol Interference (d) Intra System Interference |
| 99. | Out of the following line coding techniques, which requires highest bandwidth (a) AMI (b) Unipolar NRZ (c) Polar RZ (d) Manchester |
| 100. | The modulation process converts - the passband signal to baseband signal b) the bandpass signal to baseband signal c) the baseband signal to bandpass signal d) the passband signal to bandpass signal |

-- 00 -- 00 –