[image: image1.jpg]Sreenidhi Institute of Science and Tm:hnnlngy

AUTONOMOUS






Sreenidhi Institute of Science and Technology

(An Autonomous Institution)
Code No: 7BC56 



                 
                 Date: 19-August-2024 (AN)
B.Tech III-Year II- Semester External Examination, August - 2024 (Supplementary)
ELEMENTS OF FLUID MECHANICS AND HYDRAULIC MACHINCRY (EEE)
Time:
 3 Hours







                    Max.Marks:70


 
Note:    a)  No additional answer sheets will be provided.

          

b)  All sub-parts of a question must be answered at one place only, otherwise it will not be valued.

          

c)  Missing data can be assumed suitably.

Bloom's Cognitive Levels of Learning (BCLL)

	Remember
	L1
	Apply
	L3
	Evaluate
	L5

	Understand
	L2
	Analyze
	L4
	Create
	L6








Part - A 




Max.Marks:20
Answer all QUESTIONS

	
	
	BCLL
	CO(s)
	Marks

	1
	What are the units of kinematic viscosity?
	L2
	CO1
	[2M]

	2
	What are the applications of Bernoulli’s equation?
	L2
	CO2
	[2M]

	3
	Define hydrodynamic force.
	L1
	CO3
	[2M]

	4
	What is the need for dimensional analysis?
	L2
	CO4
	[2M]

	5
	Sketch the reciprocating pump and label the components.
	L1
	CO5
	[2M]

	6
	Define the specific speed of the turbine.
	L1
	CO6
	[2M]

	7
	What are the applications of Bernoulli’s equation?
	L2
	CO1
	[2M]

	8
	What is Couette flow and Poiseuille flow?
	L2
	CO3
	[2M]

	9
	Define Cavitation.
	L1
	CO5
	[2M]

	10
	Define Equipotential line.


	L1
	CO6
	[2M]








Part – B 




Max.Marks:50
ANSWER ANY FIVE QUESTIONS. EACH QUESTION CARRIES 10 MARKS.
	
	 
	
	BCLL
	CO(s)
	Marks

	11.
	a)
	Write short notes on differential manometers.
	L1
	CO1
	[5M]

	
	b)
	The pressure outside the droplet of water of diameter 0.04mm is 10.32N/cm2 (atmospheric Pressure). Calculate the pressure within the droplet if surface tension is given as 0.0725 N/m of Water.
	L3
	CO1
	[5M]

	
	
	
	
	
	

	12.
	a)
	State different types of fluid flows.
	L4
	CO2
	[5M]

	
	b)
	Derive the continuity equation for one dimensional flow.
	L3
	CO2
	[5M]

	
	
	
	
	
	

	13.
	a)
	Derive the expression for displacement thickness.
	L3
	CO3
	[5M]

	
	b)
	A laminar flow is taking place in a pipe of diameter 200 mm. The maximum velocity is 1.5 m/s. Find the mean velocity and the radius at which this occurs. Also calculate the velocity at 4 cm from the wall of the pipe.   
	L4
	CO3
	[5M]

	
	
	
	
	
	

	14.
	
	Explain the Dimensional analysis by using PI-Buckingham’s theorem.
	L3
	CO4
	[10M]

	
	
	
	
	
	

	15.
	a)
	What are the different efficiencies involved in the centrifugal pumps and explain them.
	L2
	CO5
	[5M]

	
	b)
	Explain the working of centrifugal pump with a neat sketch.


	L4
	CO5
	[5M]

	
	
	
	
	
	

	16.
	a)
	Explain the working of Pelton wheel with a neat sketch.
	L3
	CO6
	[5M]

	
	b)
	A turbine is operated under a head of 25m and 200rpm.The discharge is 9m3/sec if the overall efficiency is 90%. Determine the Power generated and Specific speed of the turbine.
	L4
	CO6
	[5M]

	
	
	
	
	
	

	17.
	a)
	Write short notes on differential manometers.
	L2
	CO1
	[4M]

	
	b)
	The Stream function for a two dimensional flow is given by ѱ = 2xy, calculate the velocity at the point P (2, 3). Find the velocity potential function Φ.
	L2
	CO2
	[3M]

	
	c)
	Mention the relation ship between Reynolds number and friction factor in Haigen poisseulle flow.
	L2
	CO3
	[3M]

	
	
	
	
	
	

	18.
	a)
	What is do you mean by fundamental units and derived units?
	L2
	CO4
	[4M]

	
	b)
	What are the different losses involved in the centrifugal pumps?
	L2
	CO5
	[3M]

	
	c)
	What are the different types of draft tubes, sketch them?
	L2
	CO6
	[3M]


-- 00 -- 00 –
H.T No





Regulations:


A18











PAGE  
Page 1 of 2

