[image: image1.jpg]Sreenidhi Institute of Science and Tm:hnnlngy

AUTONOMOUS






Sreenidhi Institute of Science and Technology

(An Autonomous Institution)
Code No: 7B514 




                 
      Date: 14-August-2024 (AN)
B.Tech III-Year I- Semester External Examination, August - 2024 (Supplementary)
APPLIED THERMODYNAMICS-II (MECHANICAL)
Time:
 3 Hours







                    Max.Marks:70


 
Note:    a)  No additional answer sheets will be provided.

          

b)  All sub-parts of a question must be answered at one place only, otherwise it will not be valued.

          

c)  Missing data can be assumed suitably.

Bloom's Cognitive Levels of Learning (BCLL)

	Remember
	L1
	Apply
	L3
	Evaluate
	L5

	Understand
	L2
	Analyze
	L4
	Create
	L6








Part - A 




Max.Marks:20
Answer all QUESTIONS

	
	
	BCLL
	CO(s)
	Marks

	1
	Compare Rankine and Carnot cycles.
	L2
	CO1
	[2M]

	2
	What is the function of nozzle?  What are different types of nozzles?
	L2
	CO2
	[2M]

	3
	Differentiate between Impulse and Reaction turbines.
	L2
	CO3
	[2M]

	4
	What are the sources of air leakage in condensers?
	L2
	CO4
	[2M]

	5
	How do you classify jet propulsion?
	L3
	CO5
	[2M]

	6
	Define C.O.P of a vapour compression Refrigeration system.
	L1
	CO6
	[2M]

	7
	Define Boiler horse power and efficiency.
	L1
	CO1
	[2M]

	8
	What is the criterion to decide nozzle shape?
	L3
	CO3
	[2M]

	9
	What are the effects of air leakage?
	L2
	CO4
	[2M]

	10
	Differentiate boiler mountings and accessories.
	L1
	CO2
	[2M]








Part – B 




Max.Marks:50
ANSWER ANY FIVE QUESTIONS. EACH QUESTION CARRIES 10 MARKS.
	
	 
	
	BCLL
	CO(s)
	Marks

	11.
	a)
	Draw Reheat cycle on P-V and T-S diagrams.
	L3
	CO1
	[5M]

	
	b)
	In a steam power cycle, the steam supply is at  15 bar and dry-saturated.  The condenser pressure is 0.4 bar. Calculate the Carnot and Rankine efficiencies of the cycle. Neglect pump work.
	L4
	CO1
	[5M]

	
	
	
	
	
	

	12.
	
	Determine the mass flow rate of steam through a nozzle having isentropic flow through it. Steam enters nozzle at 10 bar, 500oC and leaves at 6 bar. Cross-section area at exit of nozzle is 20 cm2.  Velocity of steam entering nozzle is negligible.
	L5
	CO2
	[10M]

	
	
	
	
	
	

	13.
	
	In a single stage impulse turbine the isentropic enthalpy drop of 200 kJ/kg occurs in the nozzle having efficiency of 96% and nozzle angle of 15°. The blade velocity coefficient is 0.96 and ratio of blade speed to steam velocity is 0.5. The steam mass flow rate is 20 kg/s and velocity of steam entering is 50 m/s. Determine, i) the blade angles at inlet and outlet if the steam enters blades smoothly and leaves axially, 

ii) the blade efficiency, 

iii) the power developed in kW, 

iv) the axial thrust.
	L5
	CO3
	[10M]

	
	
	
	
	
	

	14.
	
	A gas turbine plant has air being supplied at 1 bar, 27ºC to compressor for getting compressed upto 5 bar with isentropic efficiency of 85%. Compressed air is heated upto 1000 K in combustion chamber where also occurs a pressure drop of 0.2 bar. Subsequently expansion occurs to 1 bar in turbine.  Determine isentropic efficiency of turbine, if thermal efficiency of plant is 20%.  Neglect the air property variation throughout cycle. Take       (= 1.4
	L3
	CO4
	[10M]

	
	
	
	
	
	

	15.
	a)
	Explain the working of Ramjet engine with a neat sketch.
	
	CO5
	[5M]

	
	b)
	Explain the working of Turbo jet engine with a neat sketch.
	
	CO5
	[5M]

	
	
	
	
	
	

	16.
	a)
	List the components and explain the working principle of a ammonia vapour absorption system.
	L3
	CO6
	[5M]

	
	b)
	Define the following :     

 i) DBT       ii) WBT      iii) DPT    iv) Specific Humidity    v) Relative Humidity.
	L4
	CO6
	[5M]

	
	
	
	
	
	

	17.
	a)
	Explain different methods to increase efficiency of Rankine cycle
	L3
	CO1
	[5M]

	
	b)
	What do you understand by critical pressure ratio and chocked flow?
	L3
	CO2
	[5M]

	
	
	
	
	
	

	18.
	a)
	Explain velocity compounding with neat sketches.
	L3
	CO3
	[5M]

	
	b)
	What are the advantages and disadvantages gas turbine over I.C. Engines.
	L2
	CO4
	[5M]


-- 00 -- 00 –
H.T No





Regulations:


A18











PAGE  
Page 1 of 2

