[image: image8.jpg]Sreenidhi Institute of Science and Tm:hnnlngy

AUTONOMOUS






Sreenidhi Institute of Science and Technology

(An Autonomous Institution)
Code No: 7BC03  





                 
      Date: 30-Jan-2025 (FN)
B.Tech I-Year I- Semester External Examination, January-2025 (Supplementary)
ENGINEERING MECHANICS (ME)
Time:
 3 Hours







                    Max.Marks:70

 
Note:    a)  No additional answer sheets will be provided.

          

b)  All sub-parts of a question must be answered at one place only, otherwise it will not be valued.

          

c)  Missing data can be assumed suitably.





Bloom's Cognitive Levels of Learning (BCLL)

	Remember
	L1
	Apply
	L3
	Evaluate
	L5

	Understand
	L2
	Analyze
	L4
	Create
	L6









Part - A 



Max.Marks:20


[image: image1.wmf]Answer all QUESTIONS.
	
	
	BCLL
	CO(s)
	Marks

	1
	Determine the magnitude of the two forces such that if they act at right angles, their resultant is  10N. But if they act at 60ᵒ their resultant is 13N.
	L4
	CO1
	[2M]

	2
	What are the assumptions in developing the theory of perfect frames.
	L1
	CO2
	[2M]

	3
	State and explain pappu's theorem-I
	L1
	CO3
	[2M]

	4
	State Perpendicular axis theorem.
	L1
	CO4
	[2M]

	5
	Compare and contrast the rectilinear and curvilinear motions.



	L3
	CO5
	[2M]

	6
	State and explain work-energy theorem.
	L1
	CO6
	[2M]

	7
	List some supports in structures and mention their reactions in free body diagrams.
	L1
	CO1
	[2M]

	8
	State and explain the principle of Virtual work.
	L1
	CO2
	[2M]

	9
	Distinguish between impulse and momentum?






	L3
	CO4
	[2M]

	10
	What are the steps followed while drawing free body diagrams?
	L2
	CO5
	[2M]








           Part – B


   
 Max.Marks:50

ANSWER ANY FIVE QUESTIONS. EACH QUESTION CARRIES 10 MARKS.
	
	 
	
	BCLL
	CO(s)
	Marks

	11.
	
	Consider the 75 kg crate shown in the Fig.1. This crate was lying between two buildings and it is now being lifted onto a truck, which will remove it. The crate is supported by a vertical cable, which is joined at A to two ropes which pass over pulleys attached to the buildings at B and C. Determine the tension in each of the ropes AB and AC.
[image: image2.png]


Fig.1
	L5
	CO1
	[10M]

	
	
	
	
	
	

	12.
	
	Find the forces in all the members of the truss shown in Fig.2

[image: image3.png]E

Sm

2kN

¥m





Fig.2
	L5
	CO2
	[10M]

	
	
	
	
	
	

	13.
	
	The gravity wall is made of concrete which is shown in Fig.3. Determine the location ([image: image5.png]


 ) of the center of mass G for the wall.

[image: image6.png]


Fig.3
	L5
	CO3
	[10M]

	
	
	
	
	
	

	14.
	a)
	Derive the equation for moment of inertia of circular plate of radius R and thickness t about the centroidal axis.
	L4
	CO4
	[5M]

	
	b)
	Derive the expression for mass moment of inertia of a right circular cylinder.
	L4
	CO4
	[5M]

	
	
	
	
	
	

	15.
	a)
	Discuss the rigid body translation.
	L3
	CO5
	[5M]

	
	b)
	Two stones A and B are projected from the same point at inclinations of 450 and 300 respectively to the horizontal. Find the ratio of the velocities of projection of A and B if the maximum height reached by them is the same.
	L5
	CO5
	[5M]

	
	
	
	
	
	

	16.
	
	Determine the acceleration of the system of blocks shown in Fig.4 Assume the pulleys to be massless and frictionless.
 Take m1 = 8kg, m2= 2 kg and m3= 3 kg. Also, determine the tension in the strings connecting the blocks.
[image: image7.emf]
Fig.4
	L5
	CO6
	[10M]

	
	
	
	
	
	

	17.
	
	Explain about:
	
	
	

	
	a)
	Method of joints.
	L2
	CO1
	[5M]

	
	b)
	The working principle of screw jack.
	L2
	CO2
	[5M]

	
	
	
	
	
	

	18.
	a)
	State and prove

(i) Perpendicular axis theorem

(ii) Parallel axis theorem of moment of inertia
	L1
	CO4
	[4M]

	
	b)
	Express the acceleration of a particle in tangential and normal components.
	L3
	CO5
	[3M]

	
	c)
	State the advantage of energy and momentum methods in the analysis of plane motion of rigid bodies
	L2
	CO6
	[3M]


-- 00 -- 00 –
H.T No





Regulations:


A18











PAGE  
Page 2 of 2

_1604898669.unknown

