H.T No

Regulations:

A20

Sreenidhi Institute of Science and Technology

(An Autonomous Institution)

Code No: 8B393   					                 	  Date: 07-Feb-2025 (FN)

B.Tech II-Year I- Semester External Examination, Jan/Feb - 2025 (Supplementary)

COMPREHENSIVE TEST AND VIVA VOCE-III  (ME)

Time:	3:00 Hours								                    Max.Marks:70

Note: Answer Any 70 Questions.

Student can solve more than 70 questions, but maximum 70 marks will be awarded.

Calculator's are not allowed.

| S.No | Question |
| --- | --- |
| 1. | A gas turbine power plant uses a) Otto cycle b) Rankine cycle c) Brayton cycle d) Diesel cycle |
| 2. | The Brayton cycle consists a) two reversible isotherms and two reversible isobars b) two reversible isochores and two reversible adiabatics c) two reversible isotherms and two reversible isochores d) two reversible isobars and two reversible adiabatics |
| 3. | The available energy of a system ___ as its temperature or pressure decreases and approaches that of the surroundings a) remains constant b) increases c) decreases d) none of the mentioned |
| 4. | Heat flow into a system is ____, and heat flow out of the system is ________ a) positive, positive b) negative, negative c) negative, positive d) positive, negative |
| 5. | The entropy of an isolated system can never ____ a) decrease b) be zero c) increase d) none of the mentioned |
| 6. | What is the value of the absolute thermodynamic temperature scale? a) 3K b) 0K c) 1K d) 4K |
| 7. | Which of the following causes irreversibility a) electrical resistance, magnetic hysteresis b) friction, viscosity c) inelasticity d) all of the mentioned |
| 8. | Which of the following occurs without a change in the internal energy a) Isochoric process b) Isothermal process c) isobaric process d) Isenthalpic process |
| 9. | If heat Q flows reversibly from the system to the surroundings at To, what will be the effect on entropy a) entropy of the system is reduced by Q/To b) system has lost entropy to the surroundings c) entropy increase of the surroundings is Q/To d) all of the mentioned |
| 10. | Which of the following is a thermodynamics law a) Zeroth law of thermodynamics b) Faraday’s Law of thermodynamics c) Ideal Gas Law of thermodynamics d) Boyle’s Law of thermodynamics |
| 11. | Which of the following is true according to Clausius statement a) it is possible to construct a device that can transfer heat from a cooler body to a hotter body without any effect b) it is impossible to construct a device that can transfer heat from a cooler body to a hotter body without any effect c) it is impossible to construct a device that can transfer heat from a hotter body to a cooler body without any effect d) none of the mentioned |
| 12. | Which of the following is a branch of thermodynamics a) Equilibrium thermodynamics b) Classical thermodynamics c) Chemical thermodynamics d) All of the mentioned |
| 13. | Which of the following properties describe entropy a) point function, intensive property b) point function, extensive property c) path function, extensive property d) path function, intensive property |
| 14. | What is thermodynamics a) study of the relationship between heat and other forms of energy b) study of the conversion of chemical energy to other forms of energy c) study of the relationship between mechanical energy to other forms of energy d) study of the conversion of mechanical energy to other forms of energy |
| 15. | The enthalpy and internal energy are the function of temperature for a) all gases b) steam c) water d) ideal gas |
| 16. | The path followed in a vapour power cycle is a) boiler-condenser-turbine-pump b) boiler-turbine-condenser-pump c) boiler-turbine-pump-condenser d) boiler-pump-turbine-condenser |
| 17. | Which of the following is an application of thermodynamics a) Refrigerators b) Gas compressors c) Power plants d) All of the mentioned |
| 18. | In which of the following systems does mass transfer occur across the system boundary a) isolated system b) closed system c) open system d) none of the mentioned |
| 19. | What is the relationship between Kelvin-Planck’s and Clausius’ statements a) violation of one doesn’t violate the other b) not connected to each other c) virtually two parallel statements of the second law d) none of the mentioned |
| 20. | Which of the following is a type of thermodynamic system? a) Open system b) Closed system c) Thermally isolated system d) All of the mentioned |
| 21. | When more than one fluid stream enters or leaves the control volume, which of the following type of balance is taken a) mass balance b) energy balance c) mass balance and energy balance d) none of the mentioned |
| 22. | Which of the following thermodynamic law gives the concept of enthalpy a) First law of thermodynamics b) Second law of thermodynamics c) Third law of thermodynamics d) Fourth law of thermodynamics |
| 23. | In which of the following state does water exist? a) saturated solid state b) saturated liquid state c) saturated vapour state d) all of the mentioned |
| 24. | Which of the following follows the Carnot theorem a) Heat engines b) Gas turbine engines c) Gas compressors d) All of the mentioned |
| 25. | The entropy change of a system between two identifiable equilibrium state is ___ when the intervening process is reversible or change of state is irreversible a) depends on the process b) different c) same d) none of the mentioned |
| 26. | Which of the following is known as the inequality of Clausius a) cyclic integral of dW/T<=0 b) cyclic integral of dW/T>=0 c) cyclic integral of dQ/T>=0 d) cyclic integral of dQ/T<=0 |
| 27. | A saturation state is a state from which a change of phase may occurs a) without a change of pressure or temperature b) with a change of pressure or temperature c) both of the mentioned d) none of the mentioned |
| 28. | Cauchy –Riemann equation in polar co-ordinates are (a) ur=rvθ and vr =-ruθ (b) ur=1rvθ and vr =-1ruθ (c) ur=1rvθ and vr =-ruθ (d) ur=rvθ and vr =-1ruθ |
| 29. | If f z=ay x i(bx cy) analytic then a, b, c equals to (a) c=1 and a=-b (b) a=1 and c=-b (c) b=1 and a=-c (d) a=b=c=1 |
| 30. | The value of 1z2+4dz where c:z-2i=1 is will be a) 0 b) 15 c) π2 d) π3 |
| 31. | Expand the function 1z+1 in Taylors series about z=1 a)-121-12z-1+122z-12+… b) 121-12z-1+122z-12+… c) 121+12z-1+122z-12+… d) None |
| 32. | The sum of residues at its poles of 1z(z-1)2 is a) 1 b) 0 c) -1 d) None |
| 33. | Standard Error of sample mean for large samples is ________________ (a) (b) (c) (d) |
| 34. | A population consists of numbers 2,3,4,5 then its Population.Mean = ___________ (a) 1.5 (b) 3.5 (c) 2 (d) 4 |
| 35. | For large samples population mean = ______________ (a) E(x) (b) E() (c) Var (1) (d) Var () |
| 36. | A statistic is said to be __________________ estimation, if E (statistic) = Parameter. (a) Biased (b) Un-biased (c) Consistent (d) Inconsistent |
| 37. | The Confidence interval for Population mean µ at 95% are ________________ (a) 2.58 (b) 1.96 (c) 2.33 (d) 1.65 |
| 38. | Use of statistical techniques to industrial products is helpful in a) Judging conference or non-conference of the products b) improving quality standards c) maintaining quality standards d) maintaining quality and improving quality standards |
| 39. | The number of control charts for variables. a) 1 b) 2 c) 3 d) 4 |
| 40. | The number of control charts for attributes. a) 1 b) 2 c) 3 d) 4 |
| 41. | What is the central limit for C chart. a) C b) Mean of C c) X d) Mean of X |
| 42. | What is the central limit for Range chart. a) C b) Mean of C c) R d) Mean of R |
| 43. | In testing independence of attribute using φ² we compute expected frequency using ei = ____________ (a) Row total (b) Column total (c) Row total x Column total Grand total Grand total Grand total (d) Row total + Column Total Row total |
| 44. | In applying φ² - test the sum of frequencies should be at least = ________ a)50 (b) 30 (c) 60 (d)80 |
| 45. | In φ² - test the df without estimating population parameters from sample statistics is r is a) n (b) k-1 (c) K (d) k + 1 |
| 46. | If there are M number of Population Parameters from sample statistics then r is (a) K-1 (b) K (c) K+1 (d) K-1-M |
| 47. | To test the independence of attributes we use ____ test (a) φ² – test (b) test for specified Means (c) test diff. of 2 Means (d) Paired t – test |
| 48. | What is the formula for Price Elasticity of Demand? (a) % of change in the Price (b) % of change in the Demand % of change in the Demand % of change in the Income (c) % of change in the Demand (d) % of change in the Demand of ‘X’ % of change in the Price % of change in the Price of ‘Y’ |
| 49. | Which is a tabular form that shows quantitative relationship between price and quantitative demand? a) Law of demand b) Demand schedule c) Law of supply d) Elasticity of Demand |
| 50. | The responsiveness change in quantity of demand with responsiveness change in price is called? a) Cross elasticity b) Income elasticity c) Price elasticity d) Advertisement elasticity |
| 51. | Demand curve always slopes ---- from left to right? a) Upward b) Horizontal c) downward d) All |
| 52. | In which market, single market price prevails for the commodity (a) Monopoly market (b) Oligopoly market (c) Perfect competition market (d) Duopoly market |
| 53. | In which there are many seller exist? (a) Perfect Competition (b) Duopoly (c) Monopoly (d) Oligopoly |
| 54. | In which type of Market the Price = AR = MR? (a) Perfect Competition (b) Duopoly (c) Monopoly (d) Oligopoly |
| 55. | In perfect competition market, seller is the? (a) Price – Maker (b) Price changer (c) Price – Taker (d) Price Dictator |
| 56. | What a business owes is called? a) Bad debts b) Profit c) Liability d) Asset |
| 57. | Depreciation is considered as? a) Profit b) Loss c) Income d) Liability |
| 58. | Land is considered as? a) Expenses b) Income c) Current Asset d) Fixed Asset |
| 59. | Bank overdraft comes under? a) Current Asset b) Current Liability c) Long term Liability d) Loss |
| 60. | Capital comes under? a) Trading A/c Debit side b) Trading A/c Credit side c) Balance sheet Under Assets d) Balance sheet Under Liabilities |
| 61. | Prepaid Expenses should be shown under? a) Assets b)Liabilities c) Trial balance d) All |
| 62. | The stock which is left at the end of the financial year called? a) opening stock b) closing stock c) Finished stock d) Semi-Finished stock |
| 63. | Provision for Bad debts treated against? a) Creditors b) Debtors c) Loans d) Purchases |
| 64. | Who proposed 14 Principles of management? a) F.W.Taylor b) Henry Fayol c) Maslow d) Peter F Drucker |
| 65. | Who is considered as the Father of scientific management? a) F.W.Taylor b) Henry Fayol c) Maslow d) Peter F Drucker |
| 66. | Authority always flows? a) Top to Bottom b) Bottom to Top c) Both A & B d) Horizontal |
| 67. | Which is occupied highest need of order? a) Basic b) Social c) self actualization d) Security |
| 68. | What is the window name when you know about your personality & others also known your personality? a) Blind spot b) Arena c) Mask d) Unconscious |
| 69. | Who coined the concept of Transactional analysis? a) Peter b) Joseph luft c) Eric Berne d) Porter |
| 70. | Which is/are Internal stressors? a) Physical Environment b) social interaction c) Daily Hassles d) mind traps |
| 71. | Which is/are the remedy (ies) for controlling stress? a) Exercise b) Pranayama c) Meditation d) All |
| 72. | Which of the following is Not a component of the cotter joint assembly a) Collar b)Spigot c) Socket d) Cotter |
| 73. | Which of the following is a characteristic of an adiabatic process a)ΔU = 0 b)W = 0 c) Q = 0 d) ΔV = 0 |
| 74. | The state of an ideal gas is changed isochorically (Constant volume) from position 1 to position 2. What is the change in the internal energy of the gas during this process A)ΔU = W (B) ΔU = Q (C) ΔU > 0 (D) ΔU = 0 |
| 75. | An ideal gas is taken from state 1 to state 2 and then to state 3. If the process 1-2 is adiabatic and 2-3 is isothermal, what is a true statement about the change in temperature and heat transferred during 1-2 (A) ΔT > 0, Q > 0 (B) ΔT < 0, Q = 0 (C) ΔT = 0, Q = 0 (D) ΔT > 0, Q < 0 |
| 76. | Why austenite γ gamma iron allows maximum solid solubility of carbon compared to delta δ iron, or alpha α iron? (A). Because interstitial void size in γ gamma iron is smaller. (B). Because interstitial void size in α alpha iron is larger (C). Because interstitial void size in δ delta iron is larger (D). Because interstitial void size in γ gamma iron is larger. |
| 77. | Which one of the following is not an invariant reaction in Fe-Fe3C system? (A). Eutectic reaction (B). Peritectic reaction (C). Eutectoid reaction. D) Isothermal reaction |
| 78. | What microstructural phases are seen in an Eutectoid steel? (A). Ludeberite consisting of Fe3C and austenite. (B). Martensite needles or platelets. (C). Pearlite consisting of alternate layers of ferrite and cementite. (D). Bainite consisting of finer size Fe3C in alpha ferrite. |
| 79. | What is a proeutectioid α ferrite? (A). A phase that has formed after the formation of pearlite. (B). A phase that has formed prior to the formation of pearlite. (C). A phase that has alternate layers of ferrite and cementite. (D). A phase that has formed after the formation of cementite. |
| 80. | Why stainless steel has high corrosion resistance than mild steel? (A). Mild steel forms a thick, dense oxide scale that prevents further oxidation. (B). Stainless steel has smooth surface which stops corrosion. (C) Stainless steel forms a dense, nonporous and adherent thin Cr2O3 oxide scale that prevents further corrosion. (D). Mild steel has high corrosion resistance than stainless steel. |
| 81. | Which the following is added to steel to increase the corrosion resistance? (A) tungsten and vanadium (B) zinc and lead (C) chromium and nickel D) Sulphur and phosphorous |
| 82. | Which quenching medium offers higher cooling rate during ‘Hardening’ treatment? (A). Oil (B). Polymer (C). Gas (D) 10% NaOH dissolved in water |
| 83. | Following stress relieving process is used after cold working of materials (A) Tempering (B) Cyaniding (C) Annealing (D) Normalizing |
| 84. | One of the purposes of ‘Tempering’ a hardened steel is: (A) To relive internal stresses, increase ductility and toughness. (B) To produce large size cementite. (C). To produce pearlite (D) To produce ludeberite |
| 85. | Dislocation in materials is ___ defect. (A) Point (B) Line (C) Plane (D) Casting defect |
| 86. | Higher hardness is obtained in: (A) Tempered martensite steel. (B) Hardened steel. (C) Normalized steel (D) Annealed steel |
| 87. | Tensile strength of steel is increased by addition of (A) Manganese (B) Sulphur (C) Phosphorous (D) Carbon |
| 88. | The purpose of ‘Hardenability’ heat treatment is: A) To measure the depth at which 50% martensite is present or Measured hardness is 50 HRC (B) To produce uniform hardness of 50 HRC across the entire cross-section of the hardened steel work piece. (C) To improve machinability of the steel workpiece (D)To produce sphere-like shaped cementite. |
| 89. | Purpose of tempering is to improve (A) ductility (B) malleability (C) hardness (D) machinability |
| 90. | TTT diagram: (A). Provides solution for easy machinability of steel work piece (B). Suggests range of temperatures to be implemented for annealing. (C). Helps determining time needed for heating a steel work piece. (D). Helps determining the time and temperature needed for completion of a phase transformation |
| 91. | A Cotter Joint is also known as a) hammer joint b)socket and spigot joint c) hub joint d) collar joint |
| 92. | A hybrid composite consists of: (A). One reinforcement in one matrix (B) Two reinforcements in two matrix materials (C). Two reinforcements in one matrix. (D) Two solid reinforcements in two liquid matrices. |
| 93. | Internal Angle of ACME thread a) 45 b) 29 c) 60 d) 55 |
| 94. | Which one of the following is incorrect with respect to the uses of Shape Memory Alloys (SMA): (A) Memory effect pulls bones together to promote healing (B) SMA is used as Eye glass frames (C) SMA is used as Catheters (D) SMA is used for fabricating chairs |
| 95. | Hexagonal nut corner to corner standard dimension a) 2D b) 1.5D c) 3D d) 1.5D + 3 |
| 96. | Which one of the statements is incorrect? (A). Piezoelectric ceramic produces electricity when it is subjected to stress and strain. (B). Maraging steel is one of the strongest steels used for missile body cases. (C). Maraging steel gets its strength from the formation of martensite and Ni3Al intermetallic compound. (D). A superhydrophobic surface has a contact angle 0 to 300 and water adheres to its surface. |
| 97. | The edges of a cube in isometric projection make angles of this many degrees with each other: a) 30 b) 60 c) 90 d) 120 |
| 98. | A nanomaterial will have at least one of its dimensions: (A). in the range of 1 to 100 A0 (B). in the range of 1 to 100 μm (C). in the range of 1 to 100 nm (D). in the range of 1 to 100 cm |
| 99. | Cotters tapper ratio a) 1:30 b) 1:40 c) 1:70 d)1:100 |
| 100. | Carbon nanotube has very high strength: (A). about 200 GPa (B). about 200 kg/mm2 (C). about 200 MPa (D). about 200 kg/cm2 |

-- 00 -- 00 –