H.T No

Regulations:

A20

Sreenidhi Institute of Science and Technology

(An Autonomous Institution)

Code No: 8B494   					                 	        	        Date:07-Feb-2025 (AN)

B.Tech II-Year II- Semester External Examination, Jan/Feb - 2025 (Supplementary)

COMPREHENSIVE TEST AND VIVA VOCE-IV (ME)

Time:	3:00 Hours								                    Max.Marks:70

Note: Answer Any 70 Questions.

Student can solve more than 70 questions, but maximum 70 marks will be awarded.

Calculator's are not allowed.

| S.No | Question |
| --- | --- |
| 1. | Which one of the following is the unit of mass density? a) kg/ m3 b) kg/ m2 c) kg /m d) kg /ms |
| 2. | The specific volume of a liquid is the reciprocal of a) weight density b) mass density c) specific weight d) specific volume |
| 3. | Which one of the following is the unit of specific weight? a) N /m3 b) N /m2 c) N /m d) N /mS |
| 4. | The viscosity of a fluid in motion is 1 Poise. What will be it’s viscosity (in Poise) when the fluid is at rest? a) 0 b) 0.5 c) 1 d) 2 |
| 5. | Water flows between two plates of which the upper one is stationary and the lower one is moving with a velocity V. What will be the velocity of the fluid in contact with the upper plate? a) V b) V ⁄ 2 c) 2V d) 0 |
| 6. | For compressible fluid flow in a pipe, having decrease in specific gravity what will be the effect of decrease in diameter? a) It will cause increase in velocity b) It will cause decrease in velocity c) It remains constant d) None of the mentioned |
| 7. | What is the most common assumption while dealing with fluid flow problems using continuity equation? a) Flow is assumed to be compressible b) Flow is assumed to be unsteady c) Flow is assumed to be steady d) Flow is assumed to be turbulent |
| 8. | The diameters of a pipe at the sections 1 and 2 are 8 cm and 13 cm respectively. Find the discharge through pipe if the velocity of water flowing through the pipe at section 1 is 6 m/s. Determine also the velocity at section 2 a) 2.27 m/s b) 4.54 m/s c) 1.13 m/s d) 3.25 m/s |
| 9. | The diameter of a pipe at the section 1 is 9 cm. If the velocity of water flowing through the pipe at section 1 is 4.8 m/s and section 2 is 9 m/s, Determine the area at section 2. a) 33.93 m2 b) 67.86 m2 c) 16.96 m2 d) 38.66 m2 |
| 10. | According to Bernoulli's equation (a) Z + p/w + v²/2g = constant (b) Z + p/w - v²/2g = constant (c) Z - p/w + v²/2g = constant (d) Z - p/w - v²/2g = constant |
| 11. | Stoke is the unit of A) Kinematic viscosity in C. G. S. units B) Kinematic viscosity in M. K. S. units C) Dynamic viscosity in M. K. S. units D) Dynamic viscosity in S. I. units |
| 12. | According to Darcy's formula, the loss of head due to friction in the pipe is (where f = Darcy's coefficient, l = Length of pipe, v = Velocity of liquid in pipe, and d = Diameter of pipe) A) flv²/2gd B) flv²/gd C) 3flv²/2gd D) 4flv²/2gd |
| 13. | The loss of head due to friction in a pipe of uniform diameter in which a viscous flow is taking place, is (where RN = Reynold number) a) 1/RN b) 4/RN c) 16/RN d) 64/RN |
| 14. | The continuity equation is based on the premise of- a)Law of conservation of energy b) Law of conservation of mass c) Law of conservation of momentum d) None of the mentioned |
| 15. | What is the most common assumption while dealing with fluid flow problems using continuity equation? a) Flow is assumed to be compressible b) Flow is assumed to be unsteady c) Flow is assumed to be steady d) Flow is assumed to be turbulent |
| 16. | Which among the following is not a criterion to achieve similitude a)Geometric similarity b) Kinematic similarity c) Dynamic similarity d) Conditional similarity |
| 17. | Which among the following have the same forces acting on them? a) Geometric similarity b) Kinematic similarity c) Dynamic similarity d) Conditional similarity |
| 18. | The dimensions of surface tension are a)MT-2 b) ML-1T-1 c) ML2T-2 d) ML2T2 |
| 19. | The dimensional formula of specific gravity is a)MLT-2 b) ML-1T-1 c) ML2T-2 d) dimension less |
| 20. | A similitude is a concept applicable to the testing of a)mathematical models b) physical models c) engineering models d) chemical models |
| 21. | In centrifugal pump ........... friction is produce than reciprocating pump a) Lower b) Same c) Higher d) Can't say |
| 22. | Centrifugal pump is convert........ into .............. a) Mechanical energy, kinetic energy b) Kinetic energy, mechanical energy c) Kinetic energy into hydraulic energy d) Mechanical energy into hydraulic energy |
| 23. | Centrifugal pump is suitable for a) Low pressure application b) Medium pressure application c) High pressure application d) None of these |
| 24. | Centrifugal pump efficiency is .......... than reciprocating pump. a) More b) Less c) Same d) Can't say |
| 25. | Centrifugal pump is also know (called) as .... a) Velocity pump b) Reciprocating Pump c) Vane pump d) Screw pump |
| 26. | Kaplan turbine is ______ a) Tangential flow b) Radial flow c) Axial flow d) Mixed flow |
| 27. | Francis turbine is ______ a) Tangential flow b) Radial flow c) Axial flow d) Mixed flow |
| 28. | In mixed flow turbines, the water enters the blades ____ and comes out _____ a) radially, axially b) radially, radially c) axially, radially d) axially, axially |
| 29. | The function of draft tube is to a) increase the pressure of the exiting fluid b) increase the Kinetic energy of exiting fluid c) allow the turbine to be set below tail water level d) keep pressure at runner outlet above the atmospheric pressure |
| 30. | Cavitation occurs at a) high pressure b) low pressure c) atmospheric pressure d) none of the above |
| 31. | Degrees of freedom (n) of cylindrical pair is equal to a) 0 b) 1 c) 2 d) 3 |
| 32. | Which of the following is not four bar inversion a) Beam engine b) couple rod of locomotive c) Reciprocating compressor d) watt indicator mechanism |
| 33. | Which of the following is 2ND Inversion of single slider crank mechanism a) Beam engine b) whit worh quick return mechanism c) crank and slotter lever mechanism d) Elliptical trammel |
| 34. | In a four bar chain or quadric cycle chain a) All are turning pair b) 1 turning pair & 3 sliding pairs c) 3 turning pair& 1 sliding pairs d) All are turning pair |
| 35. | Single slider crank mechanism is consist with a) All are turning pair b) 1 turning pair & 3 sliding pairs c) 3 turning pair& 1 sliding pairs d) All are turning pair |
| 36. | The purpose of a link is to a) Transmit motion b) Guide other links c) Act as a support d) All of the above |
| 37. | A simple mechanism has A )1 link b) 2 link c) 3 link d) 4 link |
| 38. | The example of higher pair is a)Belt drives b) Cam and follower c) A and B d) None of the above |
| 39. | Davis steering gear consists of a)Sliding pairs b) Turning pairs c) rolling pairs d) All of the above |
| 40. | Which steering mechanism is present days are using a) Ackerman steering b) Davis steering c) Both A and B d)None of the above |
| 41. | The condition for correct steering angle of a Davis steering gear mechanism is a) Sin α = b/c b) Cos α = b/c c) Tan α = c/2b d) cot α = c/2b |
| 42. | Which of the following are exact straight line mechanisms a) Peaucellier Mechanism b) Hart Mechanism c) Scott-russel Mechanism d) All of the above |
| 43. | Hart mechanism is consist with a) 2 turning pair b) 4 turning pair c) 6 turning pair d) only sliding pairs |
| 44. | Peaucellier mechanism is consist with a) 2 turning pair b) 4 turning pair c) 6 turning pair d) 8 turning pair |
| 45. | Which of the following is not an approximate straight line mechanism a) Peaucellier Mechanism b) Gross hopper Mechanism c) Robert Mechanism d) Tchebicheff Mechanism |
| 46. | In a radial cam, the follower moves a) in a direction perpendicular to the cam axis b) in a direction parallel to the cam axis c) in any direction irrespective of the cam axis d) along the cam axis |
| 47. | A radial follower is one a) that reciprocates in the guides b) that oscillates c) in which the follower translates along an axis passing through the cam centre of rotation. d) none of the above |
| 48. | Offet is provided to a cam follower mechanism to a) minimise the side thrust b) accelerate c) avoid jerk d) none of these |
| 49. | For low and moderate speed engines, the cam follower should move with a) uniform velocity b) simple harmonic motion c) uniform acceleration and retardation d) cycloidal motion |
| 50. | For high speed engines, the cam follower should move with (a) uniform velocity (b) simple harmonic motion (c) uniform acceleration and retardation (d) cycloidal motion |
| 51. | In a simple gear train, if the number of idle gears is odd, then the motion of driven gear will (a) be same as that of driving gear (b) be opposite as that of driving gear (c) depend upon the number of teeth on the driving gear (d) none of the above |
| 52. | The train value of a gear train is (a) equal to velocity ratio of a gear train (b) reciprocal of velocity ratio of a gear train (c) always greater than unity (d) always less than unity |
| 53. | When the axes of first and last gear are co-axial, then gear train is known as (a) simple gear train (b) compound gear train (c) reverted gear train (d) epicyclic gear train |
| 54. | In a clock mechanism, the gear train used to connect minute hand to hour hand, is (a) epicyclic gear train (b) reverted gear train (c) compound gear train (d) simple gear train |
| 55. | In a gear train, when the axes of the shafts, over which the gears are mounted, move relative to a fixed axis, is called (a) simple gear train (b) compound gear train (c) reverted gear train (d) epicyclic gear train |
| 56. | The power transmitted by a belt is maximum when the maximum tension in the belt (T) is equal to (a) TC (b) 2TC (c) 3TC (d) 4TC where TC = Centrifugal tension. |
| 57. | The velocity of the belt for maximum power is (a) 3 T m (b) 4 T m (c) 5 T m (d) 6 T m where m = Mass of the belt in kg per metre length. |
| 58. | The centrifugal tension in belts (a) increases power transmitted (b) decreases power transmitted (c) have no effect on the power transmitted (d) increases power transmitted upto a certain speed and then decreases |
| 59. | When the belt is stationary, it is subjected to some tension, known as initial tension. The value of this tension is equal to the a)tension in the tight side of the belt (b) tension in the slack side of the belt (c) sum of the tensions in the tight side and slack side of the belt (d) average tension of the tight side and slack side of the belt |
| 60. | Effect of centrifugal tension on power transmission a)No effect (b) Effects power transmission (c) Both the above (d) None of the above |
| 61. | In circular drawing process, when the depth of drawing is more than the diameter of the die, then the process is called as (A) forced drawing (B) hollow drawing (C) deep drawing (D) all of the above |
| 62. | The roll separating force between the rolling rolls will increase if (a) roll diameter increases (b) roll diameter decreases (c) number of rolls increases (d) number of rolls decreases |
| 63. | The type of force applied through die in forging is (A) Tensile force (B) Compressive force (C) Shear force (D) Any of the above |
| 64. | Which of the following open die forging operation reduces the height of a forging and increases its diameter? (A) Cogging B) Upsetting (C) Expanding (D) Hollow forging |
| 65. | The screw type mechanical press is used for the forging of a) Steel b) Copper c) Brass d) Aluminium |
| 66. | A rigid blank holder is placed over the workpiece during deep drawing operation: a)To reduce the drawing force b) To avoid formation of the wrinkles on the cup being drawn c) To prevent mechanical damage to the punch. d) To increase the friction between die and punch |
| 67. | ‘Spinning’ is a process: a)Used for producing ferrous sheets into spherical shaped products b) Used for making sheet metals c) Used for making sheet metal into tubes. d) Used for making sheet metal into a cup or cone-shaped products that are axisymmetric. |
| 68. | Embossing is a sheet metal operation: a)Used for making raised figures on sheets with its corresponding relief on the other side. b) Used for fabrication of equipment by welding, and brazing. c) Used for manufacturing of gas hollow tight products. d) Used on sheet metals which are very hard and brittle. |
| 69. | Coining is a sheet metal operation: a)Used for making rubber products. b) In which, the workpiece is held at sub-zero temperatures. c) In which, liquid metals are poured into molds. d) In which, the metal flow occurs only at the top and bottom layers of the sheet metal and not the entire volume. |
| 70. | The type of stress induced during the ‘Stretch Forming Operation’ of sheet metal is: a)shear b) Compression c) Tension d) All the above stresses. |
| 71. | American Welding Society defines the ‘Duty Cycle’ as: a ‘Non-stop duration of welding operation’. b) Duration of the ‘Pulsed current’ welding operation c) ‘The percentage of time in a 10-minute period that a welding machine can be used at its rated output without overloading’. d) the percentage of time duration of no-load in 10 minutes. |
| 72. | The ‘Constant Current Arc Welding’ (CCAW) machine: a)allows variations in arc length (gap), but the corresponding variation in voltage does not affect a range of amperage and thus maintainsa constant current. b) requires a highly skilled operator to carry out welding. c) very difficult to maintain a constant current. d) maintenance costs are high. |
| 73. | DCSPEN: ‘Direct Current Straight Polarity Electrode Negative’ welding is useful for: a)Welding jobs that do not require a sustained supply of molten pools. B) jobs that require the highest heat input, such as (i) thick workpieces, (ii) workpieces that possess high thermal conductivity. C) welding jobs that require poor weld penetration. D) welding jobs that require shallow weld beads. |
| 74. | The ‘Heat Affected Zone’,(HAZ) in a welded product exhibits: a)Lowhardness, low strength, and coarse microstructure compared to its adjacent non-HAZ regions. B) Enhanced mechanical properties. C) Decreased resistance to stress corrosion cracking. D) Decreased thermal conductivity. |
| 75. | ‘Thermit welding’ process does not require electrical energy or a gas torch to cause fusion welding, because, a)Thermit welding works on the exothermic reaction that produces large amounts of thermal energy which cause fusion welding easily. b) Thermit welding works on Endothermic reaction, thus promoting fusion welding. c) Thermit welding works on displacement reaction to cause welding. d) Thermit welding works on decomposition reaction that leads to the generation of heat. |
| 76. | What happens during the ‘Sintering’ operation? a)During sintering, the powder particles bond to each other, and shrink, thus porosity gets reduced and a strong near-net-shape product is formed. b) Particles aggregate is formed. c) Powder particles melt and join together as solid objects. d) The additive binders melt and join the powder particles. |
| 77. | How many types of plastics and their characteristics? a)Two types, namely plastics and composites. b) Two types, namely cyclable and non-recyclable. c) Two types, namely for domestic home and for chemicals d) Two types of plastics namely (i) Thermoplastics, which once produced from their raw materials, can be recycled; (ii) Thermosetting type of plastics once produced from their raw materials, can not be recycled, as they become brittle on heating and do not soften. |
| 78. | Identify the number of plastic manufacturing processes: a)Four manufacturing processes are existing for plastics (i) Injection molding, (ii) Blow molding, (iii) Compression molding, and (iv) Transfer molding. b) Almost all plastic parts are made by the “Injection molding process”. c) Two types of processes are available, namely (i) Cyclable and recyclable. d) Two types of plastics: (i) For domestic, (ii) For industry. |
| 79. | What additives are added to metal powders during blending operation and why? a)Additives will make powders impure and affect properties. b) Additives may affect the performance of PM-products in service. c) Cheap powders contain large volumes of additives. d) (i) Lubricants (Paraffin wax, Talc,..) for die wall lubrication and (ii) binders (styrene/maleic anhydride copolymers) to enhance bonding are added to metal powders and blended. |
| 80. | What are the four important advantages of PM a) (i). Small size components can be manufactured, (ii) Powders produced by chemical reduction. b) (i) High and low vapor pressure metals can be blended easily by this method, (ii) High and low melting point metals can be blended easily. c) (i) Energy efficiency, (ii) High volume component production, (iii) Uniform chemical composition, thus uniform properties, (iv) Miscible and immiscible alloy systems d) (i) Parts with controlled porosity can be produced, (ii) Cost of powder material and tooling will be a limitation of this process. |
| 81. | In which of the following is toString() method defined? a) java.lang.Object b) java.lang.String c) java.lang.util d) None |
| 82. | Identify the modifier which cannot be used for constructor. a) public b) protected c) private d) static |
| 83. | What is the process of defining two or more methods within same class that have same name but different parameters declaration? a) method overloading b) method overriding c) method hiding d) none of the mentioned |
| 84. | Which one of the following is not a Java feature? a) Object-oriented b) Use of pointers c) Portable d) Dynamic |
| 85. | What is true about private constructor? a) Private constructor ensures only one instance of a class exist at any point of time b) Private constructor ensures multiple instances of a class exist at any point of time c) Private constructor eases the instantiation of a class d) Private constructor allows creating objects in other classes |
| 86. | ##### Which of the following is/are true statements? ##### a) A class can extend only one class but can implement many interfaces ##### b) An interface can extend many interfaces ##### c) An interface can implement another interface ##### d) An interface can implement a class |
| 87. | Which of this keyword can be used in a subclass to call the constructor of superclass? a) super b) this c) extent d) extends |
| 88. | Which of these is correct way of calling a constructor having no parameters, of superclass A by subclass B? a) super(void); b) superclass.(); c) super.A(); d) super(); |
| 89. | The first statement in java source file a)import statement b)package statement c)main statement d) try{} and catch{} |
| 90. | Choose a correct statement about Java Interfaces? a) Interface contains only abstract methods by default. b) A Java class can implement multiple interfaces c) An Interface can extend or inherit another Interface. d) All the above |
| 91. | A Static method of an Interface should be accessed with _____ and a DOT operator. a) Class Name b) Interface Name c) An object of a concrete class d) None of the above |
| 92. | Which of the following does not deal with exceptions? a)throws b) throw c)finalize d) finally |
| 93. | Which class is base class for all exceptions? a)String b) Error c)Throwable d) RuntimeException |
| 94. | Which exception is thrown by read() method ? a) Exception b) FileNotFoundException c) ReadException d) IOException |
| 95. | Which package provides many event classes and Listener interfaces for event handling? A. java.awt B. java.awt.Graphics C. java.awt.event D. None of the above |
| 96. | By which method You can set or change the text in a Label? A. setText() B. getText() C. Both A & B D. None of the above |
| 97. | Which are passive controls that do not support any interaction with the user? A. Choice B. List C. Label D. Checkbox |
| 98. | Which is a special type of program that is embedded in the webpage to generate the dynamic content? a. Package b. Applet c. Browser d. None of the above |
| 99. | Which is the correct order of lifecycle in an applet? a. Applet is started,initialized,painted,destroyed,stopped b. Applet is painted,started,stopped,initilaized,destroyed c. Applet is initialized,started,painted,stopped,destroyed d. None of the above |
| 100. | What is a listener in context to event handling? a) A listener is a variable that is notified when an event occurs b) A listener is an object that is notified when an event occurs c) A listener is a method that is notified when an event occurs d) None of the mentioned |

-- 00 -- 00 –