[image: image7.jpg]Sreenidhi Institute of Science and Tm:hnnlngy

AUTONOMOUS






Sreenidhi Institute of Science and Technology

(An Autonomous Institution)
Code No: 6W102  





                 
     Date: 19-Jan-2019 (FN)
M.Tech I-Year I-Semester End External Examination, Jan/Feb-2019 (Regular/Supplementary)
ADVANCED FINITE ELEMENT ANALYSIS (CAD/CAM)
Time:
3 Hours







                    Max.Marks:75

 
Note:    a)  No additional answer sheets will be provided.

          

b)  All sub-parts of a question must be answered at one place only, otherwise it will not be valued.

          

c)  Missing data can be assumed suitably.








Part - A 



Max.Marks:25
Answer all QUESTIONS.
	1.
	Show the eigen value problem finite element formulation and list the methods used for solving it.
	[3M]

	2.
	Develop the transformation matrix used for conversion of space truss element co-ordinates from local to global system.
	[3M]

	3.
	Show the triangular membrane plate element with nodal degree of freedom.
	[3M]

	4.
	Develop the consistent mass matrix for a linear bar element.
	[3M]

	5.
	Differentiate between steady state and unsteady state heat transfer problems by mentioning governing differential equations.
	[3M]

	6.
	List the applications of invicid and incompressible flow problems.

	[2M]

	7.
	Write the characteristics of stiffness matrix.




	[2M]

	8.
	List the properties of eigen vectors.





	[2M]

	9.
	Write the boundary conditions in conduction heat transfer problems.
	[2M]

	10.
	Differentiate between finite element and finite difference methods.

	[2M]










Part – B


   
 Max.Marks:50
ANSWER ANY FIVE QUESTIONS. EACH QUESTION CARRIES 10 MARKS.
	11.
	
	Find the eigen values and eigen vectors of the following matrix using the Jacobi method. 
[image: image2.png]



	[10M]

	
	
	
	

	12.
	
	Develop the element stiffness matrix for space truss element in global coordinate system.
	[10M]

	
	
	
	

	13.
	
	Determine the stresses in the plate shown in figure using one triangular membrane element.
[image: image3.png]——

20mm

— |

E-205.GPa,

3 t-10mm




	[10M]

	
	
	
	

	14.
	
	The properties of the stepped beam shown in figure are:


Element 1: E=200 GPa, ρ= 7800 kg/m3, diameter=50 mm


Element 2: E=70 GPa, ρ= 2800 kg/m3, diameter=25 mm
[image: image4.png]A

Element 1

Element 2

A\

250 mm.

e

A\Y





	[10M]

	
	
	
	

	15.
	
	Discuss the heat transfer problems with radiation.
	[10M]

	
	
	
	

	16.
	
	Discuss the stream function formulation and it’s finite element solution.
	[10M]

	
	
	
	

	17.
	a)
	Find the inverse of the following matrix using the decomposition [A] = [U]T[U], where [U]=Upper triangular matrix.

[image: image6.png]1
—4|
3

-1
6
a4

5
-1
1

[41=





	[3M]

	
	b)
	Discuss briefly forced vibration response of beam.

	[3M]

	
	c)
	Discuss briefly partial differential equations of steady state fluid flow problems.
	[4M]


-- 00 -- 00 –
H.T No





Regulations:


A17











PAGE  
Page 1 of 2

